C:\Documents and Settings\julia.powell\My Documents\IHO TSMAD\S100-0 main\IHO S-100 Main Oct 1 2007.doc © ISO/IEC 2007 – All rights reservedISO-IEC_ 63Complementary elementIntroductory element — Main elementÉlément introductif — Élément central — Élément complémentaireIntroductory element — Main element — Complementary elementE2007-10-2 ISO/IECISO/IEC     2007 ISO/IEC ISO/IEC _(E).        2Heading 2Heading 1    02 STD Version 2.1c20   4             INTERNATIONAL HYDROGRAPHIC ORGANIZATION

S-121 
Maritime Limits and Boundaries

# Data Classification and Encoding Guide

# Part 2
Feature Model

# Edition 1.1.0 - January 2019

Published by the

International Hydrographic Organization

4b quai Antoine 1er

Principauté de Monaco

Tel: (377) 93.10.81.00

Fax: (377) 93.10.81.40

E-mail: info@iho.int

Web: www.iho.int

© Copyright International Hydrographic Organization December 18

This work is copyright. Apart from any use permitted in accordance with the Berne Convention for the Protection of Literary and Artistic Works (1886), and except in the circumstances described below, no part may be translated, reproduced by any process, adapted, communicated or commercially exploited without prior written permission from the International Hydrographic Organization Secretariat (IHO Secretariat). Copyright in some of the material in this publication may be owned by another party and permission for the translation and/or reproduction of that material must be obtained from the owner.

This document or partial material from this document may be translated, reproduced or distributed for general information, on no more than a cost recovery basis. Copies may not be sold or distributed for profit or gain without prior written agreement of the IHO Secretariat acting for the IHO and any other copyright holders.

In the event that this document or partial material from this document is reproduced, translated or distributed under the terms described above, the following statements are to be included:

“Material from IHO publication [reference to extract: Title, Edition] is reproduced with the permission of the International Hydrographic Organization Secretariat (IHO Secretariat) (Permission No ……./…) acting for the International Hydrographic Organization (IHO), which does not accept responsibility for the correctness of the material as reproduced: in case of doubt, the IHO’s authentic text shall prevail. The incorporation of material sourced from IHO shall not be construed as constituting an endorsement by IHO of this product.”

“This [document/publication] is a translation of IHO [document/publication] [name]. The IHO has not checked this translation and therefore takes no responsibility for its accuracy. In case of doubt the source version of [name] in [language] should be consulted.” The IHO Logo or other identifiers shall not be used in any derived product without prior written permission from the IHO Secretariat.

Revision History

Changes to this document are coordinated by the IHO S-121 Project Team (S-121PT) which is a subsidiary group of the S-100 Working Group (S-100WG). New editions of this document will produced in coordination with the S-121 Product Specification, and will be made available on the IHO web site.

| Edition Number | Date | Author | Purpose |
| --- | --- | --- | --- |

Overview

Part of the product specification for S121 is a description of all the features, attributes, code lists and code list values that need to be registered in the IHO S-100 register. This document describes the various feature types and their relations and provides extensive tables describing each feature, attribute, code list and code list value in detail.

TABLE OF CONTENTS

Data Classification and Encoding Guide	i

Edition 1.1.0 - January 2019	i

Overview	ii

1	Introduction	7

2	Feature Types	7

3	Attribute Types	14

3.1	MLB Location Objects and Attributes	15

3.2	MLB Limit Objects and Attributes	16

3.3	MLB Zone Objects and Attributes	17

3.4	MLB Space Objects and Attributes	18

3.5	MLB Generic Feature Objects and Defining Attributes	18

3.6	State Specific Feature Objects and Attributes	20

4	Feature Catalogue Entries	22

4.1	S121 Location Feature Objects	22

4.1.1	Limit Point	22

4.1.2	Baseline Point	22

4.1.3	Boundary Point	23

4.1.4	Contributing Point	23

4.2	S121 Limit Feature Objects	24

4.2.1	Baseline	24

4.2.2	Normal Baseline	24

4.2.3	Straight Baseline	25

4.2.4	Archipelagic Baseline	25

4.2.5	Low Tide Elevation Baseline	26

4.2.6	Mouth of Rivers Baseline	26

4.2.7	Bay Baseline	27

4.2.8	Port Baseline	27

4.2.9	Reef Baseline	28

4.2.10	Boundary	28

4.2.11	International Boundary	29

4.2.12	Outer Limit of the Territorial Sea	29

4.2.13	Outer Limit of the Contiguous Zone	30

4.2.14	Outer Limit of the Exclusive Economic Zone	30

4.2.15	Outer Limit of the Continental Shelf	31

4.2.16	OuterLimitOfTheExclusiveEconomicZone	31

4.2.17	Outer Limit of the Roadstead	32

4.2.18	Construction Line	32

4.3	S121 Zone Feature Objects	33

4.3.1	Archipelagic Waters	33

4.3.2	Contiguous Zone	33

4.3.3	Continental Shelf	34

4.3.4	Exclusive Economic Zone	34

4.3.5	High Sea	35

4.3.6	Internal Water	35

4.3.7	Roadstead	36

4.3.8	Territorial Sea	36

4.3.9	The Area	37

4.4	S121 MLB Generic Feature Objects	37

4.4.1	Location	37

4.4.2	Limit	38

4.4.3	Zone	38

4.4.4	Space	39

4.5	S121 State Specific Objects	39

4.5.1	Outer Limit of the Inland Water	39

4.5.2	Outer Limit of the Coastal Water	40

4.5.3	Inland Water	40

4.5.4	Coastal Water	41

4.5.5	Overlap Zone	41

4.5.6	Joint Management Zone	42

4.5.7	Straight	42

4.6	S121 Information Objects	43

4.6.1	Basic Administrative Unit	43

4.6.2	Right	43

4.6.3	Responsibility	43

4.6.4	Restriction	44

4.6.5	S121 Party	44

4.6.6	S121 Group Party	45

4.6.7	Source	45

4.6.8	Governance	46

4.7	S121 Attributes	46

4.7.1	Label	46

4.7.2	Name	46

4.7.3	Legal Status Type	47

4.7.4	Context	47

4.7.5	Releasability Type	48

4.7.6	Start Lifespan	48

4.7.7	End Lifespan	48

4.7.8	Collection	49

4.7.9	Reference Number	49

4.7.10	Governance Title	49

4.7.11	Governance Description	50

4.7.12	Date Approved	50

4.7.13	Date Considered	50

4.7.14	Date Introduced	51

4.7.15	Basic Administrative Unit Name	51

4.7.16	Basic Administrative Unit Type	52

4.7.17	Basic Administrative Unit Context	52

4.7.18	Right Restriction Responsibility Description	52

4.7.19	MA Right Restriction Responsibility Share	53

4.7.20	Right Restriction Responsibility Share Check	53

4.7.21	Right Restriction Responsibility Time Specification	53

4.7.22	Right Type	54

4.7.23	Responsibility Type	54

4.7.24	Restriction Type	55

4.7.25	Party Required	55

4.7.26	Party Name	55

4.7.27	Party Role	56

4.7.28	Party Type	56

4.7.29	Party Group Type	56

4.7.30	Party Share	57

4.7.31	Original Location	57

4.7.32	Official Location	58

4.7.33	Common Location	58

4.7.34	Global Location	58

4.7.35	WGS84 Location	59

4.7.36	Volume Vertical Position	59

4.7.37	Volume Vertical Extent	59

4.7.38	Textual Position	60

4.7.39	Reference System	60

4.7.40	Interpolation Role	61

4.7.41	Location Object Type	61

4.7.42	Point Type	61

4.7.43	Limit Object Type	62

4.7.44	Arc Geometry Type	62

4.7.45	Zone Object Type	62

4.7.46	Jurisdiction Domain Type	63

4.7.47	Area Value	63

4.7.48	Reference Point	64

4.7.49	Surface Relation	64

4.7.50	Space Object Type	64

4.7.51	Volume Value	65

4.7.52	Type	65

4.7.53	Volume Size	66

4.7.54	Area Size	66

4.7.55	Source Administrative Date Stamp	66

4.7.56	Source Authoritative Date	67

4.7.57	Source Document Type	67

4.7.58	Source Availability Status	68

4.7.59	Administrative Source Type	68

4.7.60	Spatial Source Type	68

4.7.61	Source Type	69

4.7.62	Source Reference	69

4.7.63	Source Quality	69

4.7.64	Source Submission Date	70

4.7.65	Source External Archive Identifier	70

4.7.66	Source Registration Date	71

4.7.67	Source Document Name	71

4.7.68	Source Online Resource	71

4.7.69	Source Registry Number	72

4.7.70	Acceptance	72

4.7.71	Data	73

4.7.72	Extraction	73

4.7.73	Recordation	73

4.7.74	Submission	74

4.7.75	Online Resource Linkage URL	74

4.7.76	Online Resource Protocol	75

4.7.77	Online Resource Application Profile	75

4.7.78	Online Resource Name	75

4.7.79	Online Resource Description	76

4.7.80	Online Resource Function	76

4.8	S121 DataTypes	77

4.8.1	Area Value	77

4.8.2	Volume Value	77

4.8.1	Location Reference	77

4.8.2	Online Resource	78

4.9	S121 Type Lists	78

4.9.1	Location Type List	78

4.9.2	Limit Type List	79

4.9.3	Zone Type List	79

4.9.4	Space Type List	79

4.9.1	Legal Status Type List	80

4.9.2	Jurisdiction Domain Type List	80

4.9.3	Releasability Type List	81

4.9.4	Arc Geometry Type List	81

4.9.5	Spatial Source Type List	81

4.9.6	Administrative Source Type List	82

4.9.7	Party Type List	82

4.9.8	Party Group Type List	82

4.9.9	Basic Administrative Unit Type List	83

4.9.10	Legal Status Type List	83

4.9.1	Restriction Type List	83

4.9.2	Right Type List	84

4.9.3	Responsibility Type List	84

4.9.4	LA Availability Status Type	85

4.9.5	LA Surface Relation Type	85

4.9.6	LA Volume Type	85

4.9.7	LA Area Type	86

4.9.8	LA Interpolation Type	86

4.9.9	LA Point Type	86

4.9.10	CI_OnlineFunctionCode	87

4.10	S121 Listed Values	87

4.10.1	-- Legal Status Type List	87

4.10.2	-- Releasability Type List	89

4.10.3	-- LA_SurfaceRelationType	90

4.10.4	-- Basic Administrative Unit Type List	91

4.10.5	-- Location Type List	92

4.10.6	-- Limit Type List	93

4.10.7	-- Zone Type List	99

4.10.8	-- Space Type List	102

4.10.9	-- Interpolation Type List	103

4.10.10	-- Point Type List	103

4.10.11	-- Arc Geometry Type List	104

4.10.12	-- Jurisdiction Domain Type List	105

4.10.13	-- Administrative Source Type List	107

4.10.14	-- Spatial Source Type List	108

4.10.15	-- Party Type List	110

4.10.16	-- Party Group Type List	112

4.10.17	-- Restriction Type List	113

4.10.18	-- Right Type List	114

4.10.19	-- Responsibility Type List	116

4.10.20	-- LA Availability Status Type	117

4.10.21	-- LA Area Type	118

4.10.22	-- LA Volume Type	120

# Introduction

This document has been developed to provide consistent, standardized instructions for encoding S-121 data. The S121 information model is defined as a single set of data objects organized into groups.

The conceptual model defines four generic object types Location, Limit, Zone, or a Space. From these generic S121 object types any S121 object may be defined. This provides great flexibility, and allows any required feature to be developed by specializing the generic objects.

When used in a data set an object may be represented using a geometry type (Point, Curve/Line, Surface/Area). For a “Space” objects the geometric representation may be as a Surface/Area with an elevation attribute.

# Feature Types

A set of predefined feature types have been established that include the objects required for Marine Limits and Boundaries. The stereotype <MLB> (Maritime Limits and Boundaries) is used to identify the MLB Feature Types and Defining Objects.

Figure 1 illustrates the relationship of the feature types to the defining objects. The information contained in the defining objects’ attributes is included in the Feature Catalogue for each feature as applicable.

Figure 1 – MLB Features

The “location” feature types are:

Limit Point - A limit point is point on the outer limit of a maritime zone.

Baseline Point – A baseline point is a point on (i) a baseline from which the breadths of the territorial sea, contiguous zone, exclusive economic zone, and continental shelf (up to 200 nautical miles) are measured, or (ii) on an archipelagic baseline.

Boundary Point – A boundary point is a point on a boundary line between two or more States.

Contributing Point - A point, typically on the "Baseline", used for the computation of a maritime "Limit" or "Boundary" object.

The “limit” feature types are:

Boundary - A delimitation line between two or more entities, determined by agreement or through a third-party dispute settlement mechanism.

International Boundary - An international delimitation between two or more coastal States with adjacent or opposite coasts, determined by agreement or through a third-party dispute settlement mechanism.

Baseline (Articles 5, 6, 7, 9, 10, 11, 13, 14 and 47 of UNCLOS) – A baseline is a line from which the breadths of the territorial sea, contiguous zone, exclusive economic zone, and continental shelf (up to 200 nautical miles) are measured. A baseline may be determined by a combination of any of the methods provided for in articles 5, 6, 7, 9, 10, 11, 12, 13, 14 and 47 of the United Nations Convention on the Law of the Sea (UNCLOS).

Normal Baseline (Articles 5, 6 and 13 of UNCLOS) – A normal baseline is formed by joining baseline points collected along:

- the low-water line along the coast as marked on large-scale charts officially recognized by the coastal State;

- the seaward low-water line of the reef, as shown by the appropriate symbol on charts officially recognized by the coastal State, in the case of islands situated on atolls or of islands having fringing reefs; and

- the low-water line of low tide elevations which are situated at wholly or partly at a distance not exceeding the breadth of the territorial sea from the mainland or an island

Straight Baseline (Articles 7, 9 and 10 of UNCLOS) –Straight baselines are formed by joining appropriate points selected along the furthest seaward extent of the low-water line. Straight baselines can be used in localities where the coastline is deeply indented and cut into, or if there is a fringe of islands along the coast in its immediate vicinity, usually known as straight baseline turning points. According to UNCLOS:

- Where because of the presence of a delta and other natural conditions the coastline is highly unstable, the appropriate points may be selected along the furthest seaward extent of the low-water line and, notwithstanding subsequent regression of the low-water line, the straight baselines shall remain effective until changed by the coastal State in accordance with this Convention;

- The drawing of straight baselines must not depart to any appreciable extent from the general direction of the coast;

- The sea areas lying within the lines must be sufficiently closely linked to the land domain to be subject to the regime of internal waters;

- Straight baselines shall not be drawn to and from low-tide elevations, unless lighthouses or similar installations which are permanently above sea level have been built on them or except in instances where the drawing of baselines to and from such elevations has received general international recognition;

- Straight baselines may not be applied by a State in such a manner as to cut off the territorial sea of another State from the high seas or an exclusive economic zone;

- If a river flows directly into the sea, the baseline shall be a straight line across the mouth of the river between points on the low-water line of its banks;

- In the case of bays, a closing line may be drawn between the low-water marks of its natural entrance points, according to these criteria:

  - An indentation shall not be regarded as a bay unless its area is as large as, or larger than, that of the semi-circle whose diameter is a line drawn across the mouth of that indentation;

  - For the purpose of measurement, the area of an indentation is that lying between the low-water mark around the shore of the indentation and a line joining the low-water mark of its natural entrance points. Where, because of the presence of islands, an indentation has more than one mouth, the semi-circle shall be drawn on a line as long as the sum total of the lengths of the lines across the different mouths. Islands within an indentation shall be included as if they were part of the water area of the indentation;

  - If the distance between the low-water marks of the natural entrance points of a bay does not exceed 24 nautical miles, a closing line may be drawn between these two low-water marks, and the waters enclosed thereby shall be considered as internal waters.

  - Where the distance between the low-water marks of the natural entrance points of a bay exceeds 24 nautical miles, a straight baseline of 24 nautical miles shall be drawn within the bay in such a manner as to enclose the maximum area of water that is possible with a line of that length.

Archipelagic baseline (Article 47 of UNCLOS) - Archipelagic baselines are formed by joining the outermost points of the outermost islands and drying reefs of the archipelago. According to the Convention:

- Archipelagic baselines can be drawn by an Archipelagic States provided that within such baselines are included the main islands and an area in which the ratio of the area of the water to the area of the land, including atolls, is between 1 to 1 and 9 to 1;

- The length of such baselines shall not exceed 100 nautical miles, except that up to 3 per cent of the total number of baselines enclosing any archipelago may exceed that length, up to a maximum length of 125 nautical miles.

- The drawing of such baselines shall not depart to any appreciable extent from the general configuration of the archipelago.

- Such baselines shall not be drawn to and from low-tide elevations, unless lighthouses or similar installations which are permanently above sea level have been built on them or where a low-tide elevation is situated wholly or partly at a distance not exceeding the breadth of the territorial sea from the nearest island.

- The system of such baselines shall not be applied by an archipelagic State in such a manner as to cut off from the high seas or the exclusive economic zone the territorial sea of another State.

- If a part of the archipelagic waters of an archipelagic State lies between two parts of an immediately adjacent neighbouring State, existing rights and all other legitimate interests which the latter State has traditionally exercised in such waters and all rights stipulated by agreement between those States shall continue and be respected.

- For the purpose of computing the ratio of water to land under paragraph l, land areas may include waters lying within the fringing reefs of islands and atolls, including that part of a steep-sided oceanic plateau which is enclosed or nearly enclosed by a chain of limestone islands and drying reefs lying on the perimeter of the plateau.

Low-Tide Elevation Baseline (Article 13 of UNCLOS) – A low-tide elevation is a naturally formed area of land which is surrounded by and above water at low tide but submerged at high tide. Where a low-tide elevation is situated wholly or partly at a distance not exceeding the breadth of the territorial sea from the mainland or an island, the low-water line on that elevation may be used as the baseline for measuring the breadth of the territorial sea.

Mouth of Rivers Baseline (Article 9 of UNCLOS) – If a river flows directly into the sea, the baseline shall be a straight line across the mouth of the river between points on the low-water line of its banks.

Reef Baseline (Article 6 of UNCLOS) – In the case of islands situated on atolls or of islands having fringing reefs, the baseline for measuring the breadth of the territorial sea is the seaward low-water line of the reef, as shown by the appropriate symbol on charts officially recognized by the coastal State.

Bay Baseline (Article 10 of UNCLOS) – A line joining the low-water mark of its natural entrance points of a bay. A bay is a well-marked indentation whose penetration is in such proportion to the width of its mouth as to contain land-locked waters and constitute more than a mere curvature of the coast.

Port Baseline (Article 11 of UNCLOS) – A line joining the outermost permanent harbour works which form an integral part of the harbour system are regarded as forming part of the coast. Off-shore installations and artificial islands shall not be considered as permanent harbour works.

Outer Limit of the Territorial Sea (Articles 3 and 4 of UNCLOS) – The outer limit of the territorial sea is a line computed at a distance not exceeding 12 nautical miles, measured from the baselines determined in accordance with UNCLOS, unless a boundary line with an adjacent or opposite coastal State makes does not allow to extend it up to that distance.

Outer Limit of the Contiguous Zone (Article 33 of UNCLOS) – The outer limit of the contiguous zone is a line computed at a distance not exceeding 24 nautical miles, measured from the baselines determined in accordance with UNCLOS or customary international law, unless a boundary line with an adjacent or opposite coastal State does not allow to extend it up to that distance.

Outer Limit of the Exclusive Economic Zone Limit (Article 57 of UNCLOS) –The outer limit of the exclusive economic zone is a line computed at a distance not exceeding 200 nautical miles, measured from the baselines determined in accordance with UNCLOS, unless a boundary line with an adjacent or opposite coastal State does not allow to extend it up to that distance.

Outer Limit of the Continental Shelf (Article 76 of UNCLOS) – The outer limit of the continental shelf is a line delineating the seabed and subsoil of the submarine areas that extend beyond its territorial sea throughout the natural prolongation of its land territory to the outer edge of the continental margin, or to a distance of 200 nautical miles from the baselines from which the breadth of the territorial sea is measured where the outer edge of the continental margin does not extend out to that distance.

Outer Limit of the Roadstead (Article 12 of UNCLOS) - The outer limit of a roadstead is a line delineating the seaward boundary of a roadstead.

International Boundary – An international delimitation between two or more coastal States with adjacent or opposite coasts, determined by agreement or through a third-party dispute settlement mechanism.

Construction Line – An arc of used for constructing "Limit" or "Boundary" typically linking to a "Contributing Point".

The “zone” feature types are:

Internal Waters (Article 8 of UNCLOS) – Internal waters are waters on the landward side of the baseline.

Archipelagic Waters (Article 47 of UNCLOS) – Archipelagic waters are waters enclosed by the archipelagic baselines, regardless of their depth or distance from the coast. The regime of archipelagic waters extends to the air space over the archipelagic waters, as well as to their seabed and subsoil.

Territorial Sea (Part II, Sections 1-3, of UNCLOS) – The territorial sea is a belt of sea beyond a costal States’ land territory and internal waters and, in the case of an archipelagic State, its archipelagic waters. The breadth of the territorial sea cannot exceed 12 nautical miles, measured from the baselines determined in accordance with UNCLOS, unless a boundary line with an adjacent or opposite coastal State does not allow to extend it up to that distance. The regime of the territorial sea applies to the air space over the territorial sea, as well as to its seabed and subsoil.

Contiguous Zone (Part II, Section 4, of UNCLOS) – The contiguous zone is a zone contiguous to a coastal State's territorial sea. The contiguous zone may not extend beyond 24 nautical miles from the baselines used to determine the outer limit of the territorial sea.

Exclusive Economic Zone (Part V of UNCLOS) – The exclusive economic zone, is an area beyond and adjacent to a costal State’s territorial sea. The exclusive economic zone extends from the outer limit of the territorial sea up to a distance not exceeding 200 nautical miles from the baselines from which the breadth of the territorial sea is measured, unless a boundary line with an adjacent or opposite coastal State does not allow to extend it up to that distance. Coastal states have sovereign rights for the purposes of exploring exploiting, conserving and managing the natural resources, whether living or non-living, of the water column, seabed and subsoil.

Continental Shelf (Part VI of UNCLOS)  – The continental shelf is a zone of a coastal State that comprises the seabed and subsoil of the submarine areas that extend beyond its territorial sea throughout the natural prolongation of its land territory to the outer edge of the continental margin, or to a distance of 200 nautical miles from the baselines from which the breadth of the territorial sea is measured where the outer edge of the continental margin does not extend out to that distance. The regime of the continental shelf applies only to its seabed and subsoil.

Roadstead (Article 12 of UNCLOS) - Roadsteads which are normally used for the loading, unloading and anchoring of ships, and which would otherwise be situated wholly or partly outside the outer limit of the territorial sea, are included in the territorial sea.

High Seas (Part VII of UNCLOS) – The high seas are all parts of the sea that are a zone that is not included in the exclusive economic zone, in the territorial sea or in the internal waters of a State, or in the archipelagic waters of an archipelagic State. The high seas are open to all States and no state may claim sovereignty over any part of the high seas.

The Area (Part XI of UNCLOS) – The area is the sea bed and ocean floor and subsoil thereof beyond the limits of national jurisdiction. The area is delineated by the outer limits of the continental shelf of coastal States that surrounds it. The regime of the area applies only to its seabed and subsoil.

Note: Four of the feature types already have definitions in the S-100 Feature Concept Dictionary. They have the stereotype <HYDRO>. These are: Territorial Sea Area, Contiguous Zone, Exclusive Economic Zone and Continental Shelf Area. These are cases where context is important. In the navigational context these features are shown to inform a navigator since it may have impact upon the rules of passage. However in the Maritime Limits and Boundaries context these features may carry a legal status as part of a treaty or declaration of a nation's sovereign boundary. The definitions of the features Territorial Sea Area, Contiguous Zone, Exclusive Economic Zone and Continental Shelf Area must be that as given in UNCLOS. These four feature types should be changed to the <MLB> stereotype in the S-100 Feature Concept Dictionary and have their definitions revised.  There is no problem having two uses for the same feature type as long as the context for the meaning is maintained. In the eNavigation context the MLB feature types should be referenced, or four parallel feature types should exist under the <HYDRO> stereotype in the Feature Concept Dictionary which indicate that they are derived from the four MLB feature types that have the full UNCLOS definitions and permit the full set of attributes allowed in S-121 including the Right, Restriction, Responsibility and Party attributes established by reference through Information Objects.

A set of State Specific features have also been defined. These features are optional and are not used to describe datasets intended for deposit with Secretary-General of the United Nations under UNCLOS. These State Specific features allow states to describe aspects of their national data such as internal waters. States may also define additional State Specific objects for their own use. These State Specific Features are:

Inland Waters Limit – Inland Waters Limit is a segment of line used to delineate inland waters and which represents the boundary between Internal Waters and Inland Waters.

Outer Limit of Coastal Waters - The outer limit of the coastal water is a line computed at a distance not exceeding 3 nautical miles, measured from the baselines determined in accordance with UNCLOS.

Coastal Waters - Costal water is a zone on the landward side of the Coastal Water Outer Limit extending to the shoreline.

Joint Management Zone - An area of agreed joint management between several States.

Inland Waters – An area describing waters found on the landward side of the Inland Waters limits and includes all the rivers, lakes and other navigable fresh waters within a territory.

Overlap Zone – An overlap zone is a zone resulting from an overlap of zones the computed limits of which appertain to different coastal States. The limit(s) of an overlap zone would correspond to the type(s) of limit(s) that would apply if the zone(s) were not overlapping.

Straight - A passage connecting two larger bodies of water.

# Attribute Types

The Rights, Restrictions, Responsibility and Party structure effectively provides attributes to the S121 feature types by reference.

The S121_FeatureUnit is a feature type and may take on any of the feature attributes described in the Feature Concept Dictionary. Specific attributes are bound to specific features in the product specification. Figure 2 shows S121_FeatureUnit and its attributes.

The S121_FeatureUnit also has five built-in attributes. These are:

fuID: The Feature Unit Identifier allows reference to information objects using the “Oid” (Object ID) attribute. The “Oid” comprises a unique character string and a namespace identifier which is also a unique character string. An example might be "Limit-1" where "Limit" is the namespace and "1" is the unique id within that namespace.

label: The attribute label provides a short textual identifier of the feature unit. For features registered in the Feature Context Dictionary this is the full textual Feature Type Name. This attribute is mandatory because it is used as part of the human readable textual encoding.

name: The name of the feature, i.e. the 6-character code or CamelCase code identifying the feature type (as defined in the Feature Concept Dictionary). The definition, and isAbstract of the feature are recorded in the Feature Concept Dictionary so do not need to be included in attributes.

legalStatusType: This attribute used to differentiate between the legal status of particular features.

context: This attribute describes the legal or administrative aspects of the feature object.

releasabilityType:  This attribute is optionally used to differentiate between releasability status for particular features.

In addition, there are three attributes inherited through the S121_VersionedObject structure. These are:

startLifespan: start time of a specific instance version.

endLifespan: optional end time of a specific instance version.

collection: the data set or series to which the data belongs.

Other thematic and spatial attributes are by reference.

Figure 2 –FeatureUnit Attributes for an S121_Feature

## MLB Location Objects and Attributes

The S121 MLB Location Objects may take on any of the feature attributes described in the Feature Concept Dictionary. In addition, the S121 MLB Location Objects inherit the attributes from S121_FeatureUnit: fuID, label, name, context and releasabilityType as well as the three versioning attributes, beginLifespanVersion, endLifespanVersion and collection.

The attribute locationObjectType from the codelist locationTypeList identifies which Feature Unit Limit Object type this object refers to based on the feature catalogue.

The attribute interpolationType from the codelist interpolationTypeList identifies the role of point in the structure of a "Straight" line or curve.

The attribute pointType from the codelist originTypeList identifies the type of the point.

Figure 3 shows MLB Location Objects and attributes.

Figure 3 – MLB Location Objects and Attributes

## MLB Limit Objects and Attributes

The S121 MLB Limit Objects may take on any of the feature attributes described in the Feature Concept Dictionary. In addition, the S121 MLB Limit Objects inherit the attributes from S121_FeatureUnit: fuID, label, name, context and releasabilityType and the versioning attributes, beginLifespanVersion, endLifespanVersion and collection.

The feature objects Baseline, Normal Baseline, Straight Baseline, Archipelagic Baseline, LowTideElevationBaseline, MouthOfRiverBaseline, BayBaseline, ReefBaseline, PortBaseline, Boundary, InternationalBoundary, Outer Limit of the Territorial Sea, Outer Limit of the Contiguous Zone, Outer Limit of the Exclusive Economic Zone, Outer Limit of the Continental Shelf, Outer Limit of the Roadstead, and Construction Line may optionally take on the attribute arcGeometryType from the codelist arcGeometryTypeList which identifies the type of computation used to define an arc (line).

The attribute limitObjectType from the codelist limitTypeList identifies which Feature Unit Limit Object type this object refers to based on the feature catalogue.

The Figure 4 shows MLB Limit Objects and attributes.

Figure 4 – MLB Limit Objects and Attributes

## MLB Zone Objects and Attributes

The S121 MLB Zone Objects may take on any of the feature attributes described in the Feature Concept Dictionary. In addition the S121 MLB Zone Objects inherit the attributes from S121_FeatureUnit: fuID, label, name, context and releasabilityType and the versioning attributes, beginLifespanVersion, endLifespanVersion and collection.

The attribute zoneObjectType from the codelist zoneTypeList which identifies which Feature Unit Zone Object type this object refers to based on the feature catalogue.

The S121 MLB Zone Objects may optionally take on the attribute jurisdictionDomain from the codelist jurisdictionDomainTypeList which defines the vertical juridical domain of the object.

The Figure 5 shows MLB Zone Objects and attributes.

Figure 5 – MLB Zone Objects and Attributes

## MLB Space Objects and Attributes

There no MLB Space objects defined. Any Space object will need to be generated from the generic object Space and registered in the Feature Catalogue for a particular product.

## MLB Generic Feature Objects and Defining Attributes

There are four generic feature object types, Location, Limit, Zone and Space. These objects serve as the root objects for all of the registered Feature Types. However these four objects may also be instantiated and specialized where there is a need to construct as special object type.  Figure 6 illustrates the relationship of the generic feature objects and defining attributes.

Figure 6 – MLB Generic Feature Objects and Defining Attributes

## State Specific Feature Objects and Attributes

In addition to the Marine Limit and Boundaries feature types based on UNCLOS an additional set of State Specific Feature Objects are defined. These objects allow states to define aspects of the marine space that are not part of UNCLOS, such as inland waters. These Limit and Zone types behave in the same way as any of the other MLB objects. The difference is that they are completely optional and may be used at a state’s discretion. This is shown in Figure 7.

Figure 7 – State Specific Objects and Attributes

# Feature Catalogue Entries

The objects and attributes identified for use in the S121 product specification are described in conformance with the IHO S-100 Register.

## S121 Location Feature Objects

### Limit Point

| Item Type | Feature |
| --- | --- |
| Domain | MLB |
| AlphaCode | LIMPNT |
| Name | Limit Point |
| Alias | MA_LimitPoint |
| CamelCase | LimitPoint |
| Use Type | Geographic; Administrative |
| Definition | A limit point is point on the outer limit of a maritime zone. |
| Reference |  |
| Definition Source |  |
| Similarity to Source |  |
| Int1 |  |
| S4 |  |
| Recommended Attributes | locationObjectType, pointType, interpolationType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features | "BaselinePoint", "BoundaryPoint", "ContributingPoint" |
| Remarks | A point typically associated with only one party. |
| Permitted Primitives | P |

### Baseline Point

| Item Type | Feature |
| --- | --- |
| Domain | MLB |
| AlphaCode | BASEPT |
| Name | Baseline Point |
| Alias | MA_BaselinePoint |
| CamelCase | BaselinePoint |
| Use Type | Administrative |
| Definition | A point on (i) a "Baseline" from which the breadths of the "TerritorialSea", "ContiguousZone", "ExclusiveEconomicZone", and "ContinentalShelf" (up to 200 nautical miles) are measured, or (ii) on an "ArchipelagicBaseline". |
| Reference |  |
| Definition Source |  |
| Similarity to Source |  |
| Int1 |  |
| S4 |  |
| Recommended Attributes | locationObjectType, pointType, interpolationType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features | "LimitPoint", "BoundaryPoint", "ContributingPoint" |
| Remarks | Can be any point that makes up a "Baseline". |
| Permitted Primitives | P |

### Boundary Point

| Item Type | Feature |
| --- | --- |
| Proposal Type | Addition |
| Domain | MLB |
| AlphaCode | BDNPNT |
| Name | Boundary Point |
| Alias | MA_BoundaryPoint |
| CamelCase | BoundaryPoint |
| Use Type | Administrative |
| Definition | A point on a "Boundary" line between two or more States. |
| Reference |  |
| Definition Source |  |
| Similarity to Source |  |
| Int1 |  |
| S4 |  |
| Recommended Attributes | locationObjectType, pointType, interpolationType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features | "BaselinePoint", "LimitPoint", "ContributingPoint" |
| Remarks | A point associated with more than one party. |
| Permitted Primitives | P |

### Contributing Point

| Item Type | Feature |
| --- | --- |
| Proposal Type | Addition |
| Domain | MLB |
| AlphaCode | CNTPNT |
| Name | Contributing Point |
| Alias | MA_ContributingPoint |
| CamelCase | ContributingPoint |
| Use Type | Administrative |
| Definition | A point, typically on the "Baseline", used for the computation of a maritime "Limit" or "Boundary" object. |
| Reference |  |
| Definition Source |  |
| Similarity to Source |  |
| Int1 |  |
| S4 |  |
| Recommended Attributes | locationObjectType, pointType, interpolationType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features | "BaselinePoint", "LimitPoint", "BoundaryPoint" |
| Remarks |  |
| Permitted Primitives | P |

## S121 Limit Feature Objects

### Baseline

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | BASELN |
| Name: | Baseline |
| Alias: | MA_Baseline |
| CamelCase: | Baseline |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Articles 5, 6, 7, 9, 10, 11, 13, 14 and 47; |
| IHO-S32: | 390 |
| Definition Source: | UNCLOS "CI_Citation"; IHO S-32, 5th Ed. "CI_Citation |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Baseline", "ArchipelagicBaseline", "NormalBaseline", "StraightBaseline", "LowTideElevationBaseline", "MouthOrRiverBaseline", "BayBaseline", "ReefBaseline", "PortBaseline" |
| Remarks: |  |
| Permitted Primitives: | L |

### Normal Baseline

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | NORBLN |
| Name: | Normal Baseline |
| Alias: | MA_NormalBaseline |
| CamelCase: | NormalBaseline |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Articles 5, 6 and 13 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Baseline", "ArchipelagicBaseline", "NormalBaseline", "StraightBaseline", "LowTideElevationBaseline", "MouthOrRiverBaseline", "BayBaseline", "ReefBaseline", "PortBaseline" |
| Remarks: |  |
| Permitted Primitives: | L |

### Straight Baseline

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Unspecified |
| Domain: | HYDRO |
| AlphaCode: | STSLNE |
| Name: | Straight Baseline |
| Alias: | MA_StraightBaseline |
| CamelCase: | StraightBaseline |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | IHO-S32: 390; |
| IHO-S32: | 393; |
| Definition Source: | IHO S-32, 5th Ed. "CI_Citation |
| Similarity to Source: |  |
| Int1: | N42 |
| S4: | 440.4 |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Baseline", "ArchipelagicBaseline", "NormalBaseline", "StraightBaseline", "LowTideElevationBaseline", "MouthOrRiverBaseline", "BayBaseline", "ReefBaseline", "PortBaseline" |
| Remarks: |  |
| Permitted Primitives: | L |

### Archipelagic Baseline

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | ARCBLN |
| Name: | Archipelagic Baseline |
| Alias: | MA_ArchipelagicBaseline |
| CamelCase: | ArchipelagicBaseline |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 47 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Baseline", "ArchipelagicBaseline", "NormalBaseline", "StraightBaseline", "LowTideElevationBaseline", "MouthOrRiverBaseline", "BayBaseline", "ReefBaseline", "PortBaseline" |
| Remarks: |  |
| Permitted Primitives: | L |

### Low Tide Elevation Baseline

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | LTEBLN |
| Name: | Low Tide Elevation Baseline |
| Alias: | MA_LowTideElevationBaseline |
| CamelCase: | LowTideElevationBaseline |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 13 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Baseline", "ArchipelagicBaseline", "NormalBaseline", "StraightBaseline", "LowTideElevationBaseline", "MouthOrRiverBaseline", "BayBaseline", "ReefBaseline", "PortBaseline" |
| Remarks: |  |
| Permitted Primitives: | L |

### Mouth of Rivers Baseline

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | MRVBLN |
| Name: | Mouth of Rivers Baseline |
| Alias: | MA_MouthOfRiverBaseline |
| CamelCase: | MouthOfRiverBaseline |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 9 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Baseline", "ArchipelagicBaseline", "NormalBaseline", "StraightBaseline", "LowTideElevationBaseline", "MouthOrRiverBaseline", "BayBaseline", "ReefBaseline", "PortBaseline" |
| Remarks: |  |
| Permitted Primitives: | L |

### Bay Baseline

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | BAYBLN |
| Name: | Bay Baseline |
| Alias: | MA_BayBaseline |
| CamelCase: | BayBaseline |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 10 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Baseline", "ArchipelagicBaseline", "NormalBaseline", "StraightBaseline", "LowTideElevationBaseline", "MouthOrRiverBaseline", "BayBaseline", "ReefBaseline", "PortBaseline" |
| Remarks: |  |
| Permitted Primitives: | L |

### Port Baseline

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | PRTBLN |
| Name: | Port Baseline |
| Alias: | MA_PortBaseline |
| CamelCase: | PortBaseline |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 11 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Baseline", "ArchipelagicBaseline", "NormalBaseline", "StraightBaseline", "LowTideElevationBaseline", "MouthOrRiverBaseline", "BayBaseline", "ReefBaseline", "PortBaseline" |
| Remarks: |  |
| Permitted Primitives: | L |

### Reef Baseline

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | REFBLN |
| Name: | Reef Baseline |
| Alias: | MA_ReefBaseline |
| CamelCase: | ReefBaseline |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 6 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Baseline", "ArchipelagicBaseline", "NormalBaseline", "StraightBaseline", "LowTideElevationBaseline", "MouthOrRiverBaseline", "BayBaseline", "ReefBaseline", "PortBaseline" |
| Remarks: |  |
| Permitted Primitives: | L |

### Boundary

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | BNDARY |
| Name: | Boundary |
| Alias: | MA_Boundary |
| CamelCase: | Boundary |
| Use Type: | Administrative |
| Definition: | A delimitation line between two or more entities, determined by agreement or through a third-party dispute settlement mechanism. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: |  |
| Remarks: |  |
| Permitted Primitives: | L |

### International Boundary

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | INTBND |
| Name: | International Boundary |
| Alias: | MA_InternationalBoundary |
| CamelCase: | InternationalBoundary |
| Use Type: | Administrative |
| Definition: | An international delimitation between two or more coastal States with adjacent or opposite coasts, determined by agreement or through a third-party dispute settlement mechanism. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: |  |
| Remarks: |  |
| Permitted Primitives: | L |

### Outer Limit of the Territorial Sea

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | TESLIM |
| Name: | Outer Limit Of The Territorial Sea |
| Alias: | MA_OuterLimitOfTheTerritorialSea |
| CamelCase: | OuterLimitOfTheTerritorialSea |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Articles 3 and 4 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "OuterLimitOfTheTerritorialSea", "OuterLimitOfTheContiguousZone", "OuterLimitOfTheExclusiveEconomicZone", "OuterLimitOfTheContinentalShelf", "OuterLimitOfTheRoadstead" |
| Remarks: |  |
| Permitted Primitives: | L |

### Outer Limit of the Contiguous Zone

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | CZOLIM |
| Name: | Outer Limit Of The Contiguous Zone |
| Alias: | MA_OuterLimitOfTheContiguousZone |
| CamelCase: | OuterLimitOfTheContiguousZone |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 33 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "OuterLimitOfTheTerritorialSea", "OuterLimitOfTheContiguousZone", "OuterLimitOfTheExclusiveEconomicZone", "OuterLimitOfTheContinentalShelf", "OuterLimitOfTheRoadstead" |
| Remarks: |  |
| Permitted Primitives: | L |

### Outer Limit of the Exclusive Economic Zone

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | EEZLIM |
| Name: | Outer Limit Of The Exclusive Economic Zone |
| Alias: | MA_OuterLimitOfTheExclusiveEconomicZone |
| CamelCase: | OuterLimitOfTheExclusiveEconomicZone |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 57 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "OuterLimitOfTheTerritorialSea", "OuterLimitOfTheContiguousZone", "OuterLimitOfTheExclusiveEconomicZone", "OuterLimitOfTheContinentalShelf", "OuterLimitOfTheRoadstead" |
| Remarks: |  |
| Permitted Primitives: | L |

### Outer Limit of the Continental Shelf

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | CSOLIM |
| Name: | Outer Limit Of The Continental Shelf |
| Alias: | MA_OuterLimitOfTheContinentalShelf |
| CamelCase: | OuterLimitOfTheContinentalShelf |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 76 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "OuterLimitOfTheTerritorialSea", "OuterLimitOfTheContiguousZone", "OuterLimitOfTheExclusiveEconomicZone", "OuterLimitOfTheContinentalShelf", "OuterLimitOfTheRoadstead" |
| Remarks: |  |
| Permitted Primitives: | L |

### OuterLimitOfTheExclusiveEconomicZone

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | EEZLIM |
| Name: | Outer Limit Of The Exclusive Economic Zone |
| Alias: | MA_OuterLimitOfTheExclusiveEconomicZone |
| CamelCase: | OuterLimitOfTheExclusiveEconomicZone |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 57 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "OuterLimitOfTheTerritorialSea", "OuterLimitOfTheContiguousZone", "OuterLimitOfTheExclusiveEconomicZone", "OuterLimitOfTheContinentalShelf", "OuterLimitOfTheRoadstead" |
| Remarks: |  |
| Permitted Primitives: | L |

### Outer Limit of the Roadstead

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | RDSLIM |
| Name: | Outer Limit Of The Roadstead |
| Alias: | MA_OuterLimitOfTheRoadstead |
| CamelCase: | OuterLimitOfTheRoadstead |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 12 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "OuterLimitOfTheTerritorialSea", "OuterLimitOfTheContiguousZone", "OuterLimitOfTheExclusiveEconomicZone", "OuterLimitOfTheContinentalShelf", "OuterLimitOfTheRoadstead" |
| Remarks: |  |
| Permitted Primitives: | L |

### Construction Line

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | CONLIN |
| Name: | Construction Line |
| Alias: | MA_ConstructionLine |
| CamelCase: | ConstructionLine |
| Use Type: | Administrative |
| Definition: | An arc of used for constructing "Limit" or "Boundary" typically linking to a "ContributingPoint". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: |  |
| Remarks: |  |
| Permitted Primitives: | L |

## S121 Zone Feature Objects

### Archipelagic Waters

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | ARCWTR |
| Name: | Archipelagic Water |
| Alias: | MA_ArchipelagicWaters |
| CamelCase: | ArchipelagicWater |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 47 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | zoneObjectType, jurisdictionDomainType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "TerritorialSea", "ContiguousZone", "ExclusiveEconomicZone", "ContinentalShelf", "TheArea", "Roadstead" |
| Remarks: |  |
| Permitted Primitives: | A |

### Contiguous Zone

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Unspecified |
| Domain: | HYDRO |
| AlphaCode: | CONZNE |
| Name: | Contiguous Zone |
| Alias: | MA_ContiguousZone |
| CamelCase: | ContiguousZone |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Part II, Section 4; |
| IHO-S32: | 993 |
| Definition Source: | UNCLOS "CI_Citation"; IHO S-32, 5th Ed. "CI_Citation |
| Similarity to Source: | by reference |
| Int1: | N44 |
| S4: | 440.6 |
| Recommended Attributes: | zoneObjectType, jurisdictionDomainType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "ArchipelagicWater", "TerritorialSea", "ExclusiveEconomicZone", "ContinentalShelf", "TheArea", "Roadstead" |
| Remarks: |  |
| Permitted Primitives: | A |

### Continental Shelf

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Unspecified |
| Domain: | HYDRO |
| AlphaCode: | COSARE |
| Name: | Continental Shelf |
| Alias: | MA_ContinentalShelf |
| CamelCase: | ContinentalShelf |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Part VI |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: | N 46 |
| S4: | 440.8 |
| Recommended Attributes: | zoneObjectType, jurisdictionDomainType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "ArchipelagicWater", "TerritorialSea", "ContiguousZone", "ExclusiveEconomicZone", "TheArea", "Roadstead" , "HighSea" |
| Remarks: |  |
| Permitted Primitives: | A |

### Exclusive Economic Zone

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Unspecified |
| Domain: | HYDRO |
| AlphaCode: | EXEZNE |
| Name: | Exclusive Economic Zone |
| Alias: | MA_ExclusiveEconomicZone |
| CamelCase: | ExclusiveEconomicZone |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Part V; |
| IHO-S32:1723 |  |
| Definition Source: | UNCLOS "CI_Citation"; IHO S-32, 5th Ed. "CI_Citation |
| Similarity to Source: | by reference |
| Int1: | N47 |
| S4: | 440.9 |
| Recommended Attributes: | zoneObjectType, jurisdictionDomainType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | ArchipelagicWater, "TerritorialSea", "ContiguousZone", "ContinentalShelf", "TheArea", "Roadstead" |
| Remarks: |  |
| Permitted Primitives: | A |

### High Sea

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | HIGHSE |
| Name: | High Sea |
| Alias: | MA_HighSea |
| CamelCase: | HighSea |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Part VII |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | zoneObjectType, jurisdictionDomainType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "ArchipelagicWater", "TerritorialSea", "ContiguousZone", "ExclusiveEconomicZone", "ContinentalShelf", "TheArea", "Roadstead" |
| Remarks: |  |
| Permitted Primitives: | A |

### Internal Water

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | INTWTR |
| Name: | Internal Water |
| Alias: | MA_InternalWaters |
| CamelCase: | InternalWater |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 8; |
| IHO-S32: | 2484 |
| Definition Source: | UNCLOS "CI_Citation"; IHO S-32, 5th Ed. "CI_Citation |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | zoneObjectType, jurisdictionDomainType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: |  |
| Remarks: |  |
| Permitted Primitives: | A |

### Roadstead

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | RDSZNE |
| Name: | Roadstead |
| Alias: | MA_Roadstead |
| CamelCase: | Roadstead |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Article 12 |
| Definition Source: | UNCLOS "CI_Citation" |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | zoneObjectType, jurisdictionDomainType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "ArchipelagicWater", "TerritorialSea", "ContiguousZone", "ExclusiveEconomicZone", "ContinentalShelf", "TheArea" |
| Remarks: |  |
| Permitted Primitives: | A |

### Territorial Sea

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Unspecified |
| Domain: | HYDRO |
| AlphaCode: | TESARE |
| Name: | Territorial Sea |
| Alias: | MA_TerritorialSea |
| CamelCase: | TerritorialSea |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Part II, Sections 1-3; |
| IHO-S32: | 5360 |
| Definition Source: | UNCLOS "CI_Citation"; IHO S-32, 5th Ed. "CI_Citation |
| Similarity to Source: | by reference |
| Int1: | N43 |
| S4: | 440.5 |
| Recommended Attributes: | zoneObjectType, jurisdictionDomainType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "ArchipelagicWater", "ContiguousZone", "ExclusiveEconomicZone", "ContinentalShelf", "TheArea", "Roadstead" |
| Remarks: |  |
| Permitted Primitives: | A |

### The Area

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | SBAREA |
| Name: | The Area |
| Alias: | MA_TheArea |
| CamelCase: | TheArea |
| Use Type: | Administrative |
| Definition: | See the legal reference provided. |
| Reference: | UNCLOS: Part XI; |
| IHO-S32: | 227 |
| Definition Source: | UNCLOS "CI_Citation"; IHO S-32, 5th Ed. "CI_Citation |
| Similarity to Source: | by reference |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | zoneObjectType, jurisdictionDomainType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | ArchipelagicWater, "TerritorialSea", "ContiguousZone", "ExclusiveEconomicZone", "ContinentalShelf", "Roadstead" |
| Remarks: |  |
| Permitted Primitives: | A |

## S121 MLB Generic Feature Objects

### Location

| Item Type: | Feature |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MLOCTN |
| Name: | Location |
| Alias: | MA_Location |
| CamelCase: | Location |
| Use Type: | Geographic; Administrative |
| Definition: | An object that defines and attributes a point in space. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | locationObjectType, interpolationRule, pointType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Limit", "Zone", "Space" |
| Remarks: |  |
| Permitted Primitives: | P |

### Limit

| Item Type: | Feature |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MLBLIM |
| Name: | Limit |
| Alias: | MA_Limit |
| CamelCase: | Limit |
| Use Type: | Geographic; Administrative |
| Definition: | An object that defines any limit or boundary either relating to terrestrial, marine or both environments. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Location", "Zone", "Space" |
| Remarks: |  |
| Permitted Primitives: | P,L |

### Zone

| Item Type: | Feature |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAZONE |
| Name: | Zone |
| Alias: | MA_Zone |
| CamelCase: | Zone |
| Use Type: | Geographic; Administrative |
| Definition: | An object that defines an area which is logically delimited by instances of "Limit". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | zoneObjectType, jurisdictionDomainType, referencePoint, areaValue, surfaceRelationType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Location", "Limit", "Space" |
| Remarks: |  |
| Permitted Primitives: | P,L,A |

### Space

| Item Type: | Feature |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MSPACE |
| Name: | Space |
| Alias: | MA_Space |
| CamelCase: | Space |
| Use Type: | Geographic; Administrative |
| Definition: | An object that defines a volume which is logically delimited by instances of "Zone". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | spaceObjectType, jurisdictionDomainType, referencePoint, volumeValue, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "Location", "Limit", "Zone" |
| Remarks: | A "Space" is an object of 2 dimensions with a height description located in 2 or 3 dimensional "Space". This is sometimes called 2 1/2 dimensions. A "Space" has the same geometry as a "Zone" with the attributes of vertical position. The vertical position may be explicit numerical attributes of height above a reference or a textual description. |
| Permitted Primitives: | P,L,A |

## S121 State Specific Objects

### Outer Limit of the Inland Water

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Retirement |
| Domain: | MLB |
| AlphaCode: | INLLIM |
| Name: | Outer Limit Of The Inland Water |
| Alias: | MA_OuterLimitOfTheInlandWater |
| CamelCase: | OuterLimitOfTheInlandWater |
| Use Type: | Administrative |
| Definition: | A line differentiating between "InlandWater" and "CoastalWater". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: |  |
| Remarks: |  |
| Permitted Primitives: | L |

### Outer Limit of the Coastal Water

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | CWOLIM |
| Name: | Outer Limit Of The Coastal Water |
| Alias: | MA_OuterLimitOfTheCoastalWaters |
| CamelCase: | OuterLimitOfTheCoastalWaters |
| Use Type: | Administrative |
| Definition: | A line computed at a distance not exceeding 3 nautical miles from the "Baseline". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | limitObjectType, arcGeometryType, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "OuterLimitOfTheTerritorialSea", "OuterLimitOfTheContiguousZone", "OuterLimitOfTheExclusiveEconomicZone", "OuterLimitOfTheContinentalShelf" |
| Remarks: | "OuterLimitOfTheCoastalWater" is used nationally by some nations. |
| Permitted Primitives: | L |

### Inland Water

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Retirement |
| Domain: | MLB |
| AlphaCode: | INLWTR |
| Name: | Inland Water |
| Alias: | MA_InlandlWaters |
| CamelCase: | InlandWater |
| Use Type: | Administrative |
| Definition: | An area describing Water found on the landward side of the "sOuterLimit"s and includes all the rivers, lakes and other navigable fresh Water within a territory. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | zoneObjectType, jurisdictionalDomain, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "InternalWater" |
| Remarks: |  |
| Permitted Primitives: | A |

### Coastal Water

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| AlphaCode: | COSWTR |
| Name: | Coastal Water |
| Alias: | MA_CoastalWater |
| CamelCase: | CoastalWater |
| Use Type: | Administrative |
| Definition: | A "Zone" on the landward side of the "CoastalWaterOuterLimit" extending to the "Baseline". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | zoneObjectType, jurisdictionalDomain, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "InlandWater", "InternalWater" |
| Remarks: | A "Zone" that is bounded by the "CoastalWaterOuterLimit" and the "InlandWaterOuterLimit". |
| Permitted Primitives: | A |

### Overlap Zone

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Retirement |
| Domain: | MLB |
| AlphaCode: | OVRZNE |
| Name: | Overlap Zone |
| Alias: | MA_OverlapZone |
| CamelCase: | OverlapZone |
| Use Type: | Administrative |
| Definition: | A "Zone" were the rights of more then one "Party" overlaps or are shared. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | zoneObjectType, jurisdictionalDomain, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "ArchipelagicWater", "TerritorialSea", "ContiguousZone", "ExclusiveEconomicZone", "ContinentalShelf", "TheArea", "Roadstead", "HighSea" |
| Remarks: |  |
| Permitted Primitives: | A |

### Joint Management Zone

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Retirement |
| Domain: | MLB |
| AlphaCode: | JMGZNE |
| Name: | Joint Management Zone |
| Alias: | MA_JointManagementZone |
| CamelCase: | JointManagementZone |
| Use Type: | Administrative |
| Definition: | An area of agreed joint management between several States. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | zoneObjectType, jurisdictionalDomain, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: | "ArchipelagicWater", "TerritorialSea", "ContiguousZone", "ExclusiveEconomicZone", "ContinentalShelf", "TheArea", "Roadstead" |
| Remarks: |  |
| Permitted Primitives: | A |

### Straight

| Item Type: | Feature |
| --- | --- |
| Proposal Type: | Retirement |
| Domain: | MLB |
| AlphaCode: | STRZNE |
| Name: | Straight |
| Alias: | MA_Straight |
| CamelCase: | Straight |
| Use Type: | Administrative |
| Definition: | A passage connecting two larger bodies of water. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Recommended Attributes: | zoneObjectType, jurisdictionalDomain, label, name, context and releasabilityType, beginLifespanVersion, endLifespanVersion, collection |
| Distinguishing Features: |  |
| Remarks: |  |
| Permitted Primitives: | L, A |

## S121 Information Objects

### Basic Administrative Unit

| Item Type: | Information |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | BAUNIT |
| Name: | Basic Administrative Unit |
| Alias: | MA_BasicAdministrativeUnit |
| CamelCase: | basicAdministrativeUnit |
| Definition: | An information object to which (one or more) unique and homogeneous "Right", "Responsibility" or "Restriction" are associated. It is an information object since it does not directly take on spatial attributes. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 4.1.2 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### Right

| Item Type: | Information |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MARGHT |
| Name: | Right |
| Alias: | MA_Right |
| CamelCase: | right |
| Definition: | An action, activity or class of actions that a system participant may perform on or using an associated resource. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 4.1.2 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: | Realized from ISO 19152 LADM "LA_Right" which stereotypes this object as a "featureType". Since it is not a feature in the same sense as the term feature is used in IHO S100, the stereotype has been removed. For the S121 code lists have been specialized to the marine environment. They can be referenced as objects from the attributes associated with an S100 Feature Object. Used in this way an information object such as an "Right" is a shared attribute by reference. |

### Responsibility

| Item Type: | Information |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MARSPN |
| Name: | Responsibility |
| Alias: | MA_Responsibility |
| CamelCase: | responsibility |
| Definition: | A formal or informal obligation to do something. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.5 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: | Realized from ISO 19152 LADM "LA_Responsibility" which stereotypes this object as a "featureType". Since it is not a feature in the same sense as the term feature is used in IHO S100, the stereotype has been removed. For the S121 code lists have been specialized to the marine environment. They can be referenced as objects from the attributes associated with an S100 Feature Object. Used in this way an information object such as an "Responsibility" is a shared attribute by reference. |

### Restriction

| Item Type: | Information |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MARSTN |
| Name: | Restriction |
| Alias: | MA_Restriction |
| CamelCase: | restriction |
| Definition: | A formal or informal entitlement to refrain from doing something. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.4 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: | Realized from ISO 19152 LADM "LA_Restriction" which stereotypes this object as a "featureType". Since it is not a feature in the same sense as the term feature is used in IHO S100, the stereotype has been removed. For the S121 code lists have been specialized to the marine environment. They can be referenced as objects from the attributes associated with an S100 Feature Object. Used in this way an information object such as an "Restriction" is a shared attribute by reference. Out of the RRR objects, it is the only one that does not require a relation with Party, since restrictions can apply to all. |

### S121 Party

| Item Type: | Information |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAPART |
| Name: | Party |
| Alias: | MA_Party |
| CamelCase: | party |
| Definition: | A person or organisation or group that plays a role in a rights transaction. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.3.1 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: | Realized from ISO 19152 LADM "LA_Party" which stereotypes this object as a "featureType". Since it is not a feature in the same sense as the term feature is used in IHO S100, the stereotype has been removed. LA_GroupParty describes any number of parties, forming together a distinct entity, with each party registered. Since LA_GroupParty is a subtype of LA_Party this is described as a single class "Party" that includes an conditional attribute "partyGroupTypeList" that defines the type of a group party. |

### S121 Group Party

| Item Type: | Information |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAGPRT |
| Name: | Group Party |
| Alias: | MA_GroupParty |
| CamelCase: | groupParty |
| Definition: | A group of two or more parties that plays a role in a rights transaction that plays a role in a rights transaction. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.3.2 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: | Realized from ISO 19152 LADM "LA_GroupParty" which stereotypes this object as a "featureType". Since it is not a feature in the same sense as the term feature is used in IHO S100, the stereotype has been removed. LA_GroupParty describes any number of parties, forming together a distinct entity, with each party registered. Since LA_GroupParty is a subtype of LA_Party this is described as a single class "Party" that includes an conditional attribute "partyGroupTypeList" that defines the type of a group party. |

### Source

| Item Type: | Information |
| --- | --- |
| Domain: | MLB |
| AlphaCode: |  |
| Name: | Source |
| Alias: |  |
| CamelCase: | Source |
| Definition: | Descriptive documentation that supports, complement or describes the associated object. Document providing legal and/or administrative facts on which the right, restriction, responsibility, basic administrative unit, party, or spatial unit is based. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 4.1.3.1 and 6.2.4 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: | Realized from ISO 19152 LADM "LA_Source" which stereotypes this object as a "featureType". Since it is not a feature in the same sense as the term feature is used in IHO S100, the stereotype has been removed. The "Source" objects are shared attributes by reference. |

### Governance

| Item Type: | Information |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | GOVERN |
| Name: | Governance |
| Alias: | MA_Goverance |
| CamelCase: | Governance |
| Definition: | A description of the legal context information from a proclamation, law or treaty document. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source:specialization |  |
| Int1: |  |
| S4: |  |
| Remarks: | Governance information defines title, reference number, date and other associated information linked to the particular proclamation, law and treaty documents to support the provision of specific description or interpretation that is applicable to the selected set of features. A governance object should contain all of the required legal text. |

## S121 Attributes

### Label

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAFLAB |
| Name: | Label |
| Alias: | MA_Label |
| CamelCase: | label |
| Data Type: | CharacterString |
| Definition: | A short textual identifier of the feature or information object. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | For features or information objects registered in the Feature Context Dictionary this is the full textual Feature / Information Object Type Name. This attribute is mandatory because it is used as part of the human readable textual encoding. |
| Quantity Specifications: |  |

### Name

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MANAME |
| Name: | Name |
| Alias: | MA_Name |
| CamelCase: | name |
| Data Type: | CharacterString |
| Definition: | The name of the feature or information object |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Legal Status Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MALGST |
| Name: | Legal Status Type |
| Alias: | MA_LegalStatusType |
| CamelCase: | legalStatusType |
| Data Type: | Legal Status Type List |
| Definition: | This attribute used to differentiate between the legal status of particular features. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Context

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MACONT |
| Name: | Context |
| Alias: | MA_Context |
| CamelCase: | context |
| Data Type: | CharacterString |
| Definition: | This attribute describes the legal or administrative aspects of the feature object. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Releasability Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MARELT |
| Name: | Releasability Type |
| Alias: | MA_ReleasabilityType |
| CamelCase: | releasabilityType |
| Data Type: | releasabilityTypeList |
| Definition: | This attribute is used to differentiate between releasability status for particular features or information objects. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Start Lifespan

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | STAVER |
| Name: | Start Lifespan |
| Alias: | MA_beginLifespanVersion |
| CamelCase: | startLifespan |
| Data Type: | DateTime |
| Definition: | Start time of a specific instance version. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: | The start time/date is mandatory. |
| Quantity Specifications: |  |

### End Lifespan

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | ENDVER |
| Name: | End Lifespan |
| Alias: | MA_EndLifespanVersion |
| CamelCase: | endLifespan |
| Data Type: | DateTime |
| Definition: | End time of a specific instance version. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: | The end time/date is optional and when missing indicates that the version is current. |
| Quantity Specifications: |  |

### Collection

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: |  |
| Name: | Collection |
| Alias: | MA_Collection |
| CamelCase: | collection |
| Data Type: |  |
| Definition: | This attribute identifies the data set or series to which the data belongs. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | Each nation or organization creating data needs to identify the data. It is expected that the State name should be used when depositing. |
| Quantity Specifications: |  |

### Reference Number

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | GORFNB |
| Name: | Reference Number |
| Alias: |  |
| Data Type: |  |
| CamelCase: | referenceNumber |
| Definition: | An attribute describing the reference number of the legal reference. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### Governance Title

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | GOVTIT |
| Name: | Governance Title |
| Alias: |  |
| Data Type: |  |
| CamelCase: | governanceTitle |
| Definition: | The title of the legal reference. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### Governance Description

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | GOVDES |
| Name: | Governance Description |
| Alias: |  |
| Data Type: |  |
| CamelCase: | governanceDescription |
| Definition: | A character string attribute containing the description of the governance statement. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | This attribute contains the block or blocks of legal text that often acts as a preamble to a proclamation, treaty or other legal document. |

### Date Approved

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: |  |
| Name: | Date Approved |
| Alias: |  |
| CamelCase: | dateApproved |
| Data Type: |  |
| Definition: | The date at which the legal statement or document was approved by the appropriate governing body. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Date Considered

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: |  |
| Name: | Date Considered |
| Alias: |  |
| CamelCase: | dateConsidered |
| Data Type: |  |
| Definition: | The date at which the legal statement or document was considered by the appropriate governing body. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Date Introduced

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: |  |
| Name: | Date Introduced |
| Alias: |  |
| CamelCase: | dateIntroduced |
| Data Type: |  |
| Definition: | The date at which the legal statement or document was introduced. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Basic Administrative Unit Name

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MABAUN |
| Name: | Basic Administrative Unit Name |
| Alias: | MA_BAUnitName |
| CamelCase: | basicAdministrativeUnitName |
| Data Type: |  |
| Definition: | Optional name for an instance of a "BasicAdministrativeUnit". |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.1 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Basic Administrative Unit Type

| tem Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MABAUT |
| Name: | Basic Administrative Unit Type |
| Alias: | MA_BAUnitType |
| CamelCase: | basicAdministrativeUnitType |
| Data Type: |  |
| Definition: | The use type of a "BasicAdministrativeUnit". |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.1 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Basic Administrative Unit Context

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MABAUC |
| Name: | Basic Administrative Unit Context |
| Alias: | MA_BAUnitContext |
| CamelCase: | basicAdministrativeUnitContext |
| Data Type: |  |
| Definition: | Description of the context for an instance of a "BasicAdministrativeUnit". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Right Restriction Responsibility Description

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MARRRD |
| Name: | Right Restriction Responsibility Description |
| Alias: | MA_RRRDescription |
| CamelCase: | rightRestrictionResponsibilitiesDescription |
| Data Type: |  |
| Definition: | Description regarding the "Right", "Restriction" or "Responsibility". |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.2 |
| Similarity to Source:rstyled |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### MA Right Restriction Responsibility Share

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MARSHR |
| Name: | Right Restriction Responsibility Share |
| Alias: | MA_RRRShare |
| CamelCase: | rightRestrictionResponsibilitiesShare |
| Data Type: |  |
| Definition: | A share in an instance of a subclass of a Right, Restriction or Responsibility. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.2 |
| Similarity to Source:specialization |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Right Restriction Responsibility Share Check

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MARSHC |
| Name: | Right Restriction Responsibility Share Check |
| Alias: | MA_RRRShareCheck |
| CamelCase: | rightRestrictionResponsibilitiesShareCheck |
| Data Type: |  |
| Definition: | Boolean indicating whether the constraint is applicable. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.2 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Right Restriction Responsibility Time Specification

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MARRRT |
| Name: | Right Restriction Responsibility Time Specification |
| Alias: | MA_RRRtimeSpec |
| CamelCase: | rightRestrictionResponsibilitiesTimeSpec |
| Data Type: |  |
| Definition: | Operational use of a right in time sharing. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.2 |
| Similarity to Source:specialization |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Right Type

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MARIGT |
| Name: | Right Type |
| Alias: | MA_RightType |
| CamelCase: | rightType |
| Data Type: |  |
| Definition: | The type of the right. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.3 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Responsibility Type

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MAREST |
| Name: | Responsibility Type |
| Alias: | MA_ResponsibilityType |
| CamelCase: | responsibilityType |
| Data Type: |  |
| Definition: | The type of the responsibility. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.5 |
| Similarity to Source:specialization |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Restriction Type

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MARSTT |
| Name: | Restriction Type |
| Alias: | MA_RestrictionType |
| CamelCase: | restrictionType |
| Data Type: |  |
| Definition: | The type of the restriction. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.9 |
| Similarity to Source:specialization |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Party Required

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MAPREQ |
| Name: | Party Required |
| Alias: | MA_PartyRequired |
| CamelCase: | partyRequired |
| Data Type: |  |
| Definition: | Indicates whether a party is required for the registration of the restriction. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.4 |
| Similarity to Source: | specilization |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Party Name

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAPNAM |
| Name: | Party Name |
| Alias: | MA_PartyName |
| CamelCase: | partyName |
| Data Type: | CharacterString |
| Definition: | The name of the party |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.3.1 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Party Role

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAPROL |
| Name: | Party Role |
| Alias: | MA_PartyRole |
| CamelCase: | partyRole |
| Data Type: |  |
| Definition: | The role of the party |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.3.1 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Party Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAPTYP |
| Name: | Party Type |
| Alias: | MA_PartyType |
| CamelCase: | partyType |
| Data Type: | Party Type List |
| Definition: | The type of party |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.3.1 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Party Group Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: |  |
| Name: | Party Group Type |
| Alias: | MA_PartyGroupType |
| CamelCase: | partyGroupType |
| Data Type: |  |
| Definition: | The type of "partyGroup". |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.3.2 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Party Share

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: |  |
| Name: | Party Share |
| Alias: | MA_PartyShare |
| CamelCase: | partyShare |
| Data Type: | Fraction |
| Definition: | The fraction of the whole. |
| Reference: | ISO 19152 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Original Location

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAORGL |
| Name: | Original Location |
| Alias: | MA_OriginalLocation |
| CamelCase: | originalLocation |
| Data Type: |  |
| Definition: | Reference location of a point when it was first established |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | This includes a description of the point as a textual string, so that the exact representation of the point may be maintained in terms of degrees and decimal degrees, or degrees, minutes and seconds, or other description. In the case of the original location this may be a description of meets and bounds or whatever is included in the treaty or other original document. A reference system (CRS) shall be included where available. Often a CRS is not available for original locations derived from very old treaties. |
| Quantity Specifications: |  |

### Official Location

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAOFFL |
| Name: | Official Location |
| Alias: | MA_OfficialLocation |
| CamelCase: | officialLocation |
| Data Type: |  |
| Definition: | Official reference of the point. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | This is the legal description of the point in a textual form, so that the exact representation of the point may be maintained in terms of degrees and decimal degrees, or degrees, minutes and seconds, or other description. A reference system (CRS) shall be included where available. A CRS is required, but it may only be included in the base legal text and therefore only included by reference through a Source object. |
| Quantity Specifications: |  |

### Common Location

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MACOML |
| Name: | Common Location |
| Alias: | MA_CommonLocation |
| CamelCase: | commonLocation |
| Data Type: |  |
| Definition: | Common reference location of point in a datum that is defaulted to the other points in the dataset. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Global Location

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAGLOL |
| Name: | Global Location |
| Alias: | MA_GlobalLocation |
| CamelCase: | globalLocation |
| Data Type: |  |
| Definition: | Global reference to a point in a world global datum such ITRF2000. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### WGS84 Location

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAWGSL |
| Name: | WGS84 Location |
| Alias: | MA_wgs84Location |
| CamelCase: | wgs84Location |
| Data Type: |  |
| Definition: | Reference to a point in global datum WGS84. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | This attribute is optional but it is requested for data deposited with DOALOS. |
| Quantity Specifications: |  |

### Volume Vertical Position

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAVPOS |
| Name: | Volume Vertical Position |
| Alias: | MA_VerticalPosition |
| CamelCase: | volumeVerticalPosition |
| Data Type: |  |
| Definition: | A reference point used to establish a bound for the vertical extent of a volume. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | GM_Point geometry as per IHO S100 Part 7 Spatial Schema, defining a reference from which to measure vertical extent. |
| Quantity Specifications: |  |

### Volume Vertical Extent

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAVEXT |
| Name: | Volume Vertical Extent |
| Alias: | MA_verticalExtent |
| CamelCase: | volumeVerticalExtent |
| Data Type: |  |
| Definition: | The vertical extent of a volume established as a distance measured against a vertical position reference point. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | Vertical extent is given in the units established through units of measure. |
| Quantity Specifications: |  |

### Textual Position

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MATXTP |
| Name: | Textual Position |
| Alias: | MA_TextualPosition |
| CamelCase: | textualPosition |
| Data Type: |  |
| Definition: | A point coordinate values in a textual form |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Reference System

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MALCRS |
| Name: | Reference System |
| Alias: | MA_ReferenceSystem |
| CamelCase: | referenceSystem |
| Data Type: |  |
| Definition: | The attribute "ReferenceSystem" allows a CoordinateReferencingSystem (CRS) to optionally be specified at the "spatialAttributeTypeList" level. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | In many other S100 based products the CRS is only defined at the metadata level and applies for the whole data set; however, in S121 it is necessary to detail it right down to the specific instances of geometry since treaty points and lines may come from different sources such as different treaties that may use different CRS. |
| Quantity Specifications: |  |

### Interpolation Role

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAINTP |
| Name: | Interpolation Role |
| Alias: | MA_ Interpolation Role |
| CamelCase: | interpolationRole |
| Data Type: | interpolationTypeList |
| Definition: | The role of point in the structure of a "Straight" line or curve. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.6.1 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Location Object Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MALOBT |
| Name: | Location Object Type |
| Alias: | MA_LocationObjectType |
| CamelCase: | locationObjectType |
| Data Type: | locationTypeList |
| Definition: | This attribute identifies which Feature Unit Location Object type this object refers to based on the feature catalogue. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Point Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAPNTT |
| Name: | Point Type |
| Alias: | MA_PointType |
| CamelCase: | pointType |
| Data Type: | pointTypeList |
| Definition: | The type of point. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.6.1 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Limit Object Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MALIMT |
| Name: | Limit Object Type |
| Alias: | MA_Limit Object Type |
| CamelCase: | limitObjectType |
| Data Type: | limitTypeList |
| Definition: | This attribute identifies which Feature Unit LimitObject type this object refers to based on the feature calatlogue. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Arc Geometry Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAARTP |
| Name: | Arc Geometry Type |
| Alias: | MA_ArcGeometryType |
| CamelCase: | arcGeometryType |
| Data Type: | arcGeometryTypeList |
| Definition: | Type of computation used to define an arc (line). |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Zone Object Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAZONT |
| Name: | Zone Object Type |
| Alias: | MA_ZoneObjectType |
| CamelCase: | zoneObjectType |
| Data Type: | zoneTypeList |
| Definition: | This attribute identifies which Feature Unit Zone Object type this object refers to based on the feature catalogue. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Jurisdiction Domain Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAJDOM |
| Name: | Jurisdiction Domain Type |
| Alias: | MA_JurisdictionDomainType |
| CamelCase: | jurisdictionDomainType |
| Data Type: | jurisdictionDomainTypeList |
| Definition: | Defines the vertical jurisdictional domain of the object delimited. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Area Value

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAAREA |
| Name: | Area Value |
| Alias: | MA_AreaValue |
| CamelCase: | areaValue |
| Data Type: | LA_AreaValue |
| Definition: | The area value. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.5.1 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Reference Point

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAREFP |
| Name: | Reference Point |
| Alias: | MA_ReferencePoint |
| CamelCase: | referencePoint |
| Data Type: | GM_Point |
| Definition: | The coordinates of a point inside the spatial unit |
| Reference: | ISO 19107 and S-100 |
| Definition Source: | ISO 19152:2012 clause 6.5.1 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Surface Relation

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASURR |
| Name: | Surface Relation |
| Alias: | MA_SurfaceRelation |
| CamelCase: | surfaceRelation |
| Data Type: | LA_SurfaceRelationType |
| Definition: | Relationship to surface. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.5.1 |
| Similarity to Source: | generalization |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Space Object Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASPOT |
| Name: | Space Object Type |
| Alias: | MA_SpaceObjectType |
| CamelCase: | spaceObjectType |
| Data Type: | spaceTypeList |
| Definition: | This attribute identifies which Feature Unit Space Object type this object refers to based on the feature catalogue. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Volume Value

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASUUR |
| Name: | Volume Value |
| Alias: | MA_VolumeValue |
| CamelCase: | volumeValue |
| Data Type: | LA_VolumeValue |
| Definition: | The volume of the 3D spatial unit |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.5.1 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MATYPE |
| Name: | Type |
| Alias: | MA_Type |
| CamelCase: | type |
| Data Type: | LA_VolumeType or LA_AreaType |
| Definition: | Indicates the type of area or volume provided |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.5.7 |
| Similarity to Source: | generalization |
| Int1: |  |
| S4: |  |
| Remarks: | This attribute is derived directly from ISO 19152 and is used in classes derived from ISO 19152. In S-121 this attribute is used in the two classes LA_VolumeValue and LA_AreaValue and relates to the two attribute data types LA_VolumeType and LA_AreaType respectively. Sine the term “type” is such a generic word, the meaning of this attribute must be constrained to only the S-121 MLB namespace. |
| Quantity Specifications: |  |

### Volume Size

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAVSIZ |
| Name: | Volume Size |
| Alias: | MA_VolumeSize |
| CamelCase: | volumeSize |
| Data Type: | volume |
| Definition: | Indicates the value of the volume |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.5.7 |
| Similarity to Source: | generalization |
| Int1: |  |
| S4: |  |
| Remarks: | This attribute is derived directly from ISO 19152 and is used in classes derived from ISO 19152. The datatype “volume” is derived from ISO 19103 (basic types). |
| Quantity Specifications: |  |

### Area Size

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAASIZ |
| Name: | Area Size |
| Alias: | MA_AreaSize |
| CamelCase: | areaSize |
| Data Type: | area |
| Definition: | Indicates the value of the area |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.5.7 |
| Similarity to Source: | generalization |
| Int1: |  |
| S4: |  |
| Remarks: | This attribute is derived directly from ISO 19152 and is used in classes derived from ISO 19152. The datatype “area” is derived from ISO 19103 (basic types). |
| Quantity Specifications: |  |

### Source Administrative Date Stamp

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASASD |
| Name: | Source Administrative Date Stamp |
| Alias: | MA_SourceAdministrativeDateStamp |
| CamelCase: | sourceAdministrativeDateStamp |
| Data Type: | DateTime |
| Definition: | The moment that the event represented by the instance of "Source" is further processed. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2014 clause 6.2.4 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: | This is the moment of "endLifespan" of old instances, and the moment of "startLifespan" of new instances. |
| Quantity Specifications: |  |

### Source Authoritative Date

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAACPT |
| Name: | Source Authoritative Date |
| Alias: | MA_SourceAuthoritiveDate |
| CamelCase: | sourceAuthoritiveDate |
| Data Type: | DateTime |
| Definition: | The date of force of law of the "Source" by an authority. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2014 clause 6.2.4 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Source Document Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASODT |
| Name: | Source Document Type |
| Alias: | MA_SourceDocumentType |
| CamelCase: | sourceDocumentType |
| Data Type: | CharacterString |
| Definition: | The type of document |
| Reference: | ISO 19152; ISO 19115 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | In ISO 19152 the equivalent attribute takes its value from ISO 19115 metadata from the class CI_PresentationFormCode. This part of the ISO metadata standard is not included in S100 part 4 metadata. Therefore a free form character string is provided as the data type. |
| Quantity Specifications: |  |

### Source Availability Status

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASLSS |
| Name: | Source Availability Status |
| Alias: | MA_SourceAvailabilityStatus |
| CamelCase: | sourceAvailabilityStatus |
| Data Type: | LA_AvailableStatusType |
| Definition: | The availability status of source document. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152 clause 6.2.4 |
| Similarity to Source:restyled |  |
| Int1: |  |
| S4: |  |
| Remarks: | The code list is taken directly for ISO 19152. |
| Quantity Specifications: |  |

### Administrative Source Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAASTY |
| Name: | Administrative Source Type |
| Alias: | MA_AdministrativeSourceType |
| CamelCase: | administrativeSourceType |
| Data Type: | administrativeSourceTypeList |
| Definition: | The type of administrative "Source" document |
| Reference: | ISO 19152 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Spatial Source Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASSTY |
| Name: | Spatial Source Type |
| Alias: | MA_SpatialSourceType |
| CamelCase: | spatialSourceType |
| Data Type: | spatialSourceTypeList |
| Definition: | The type of spatial "Source" document |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Source Type

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASTYP |
| Name: | Source Type |
| Alias: | MA_SourceType |
| CamelCase: | sourceType |
| Data Type: | CharacterString |
| Definition: | The type of "Source" document |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | Allows for generic sources other than "spatialSourceType" or "adminstrativeSourceType". |
| Quantity Specifications: |  |

### Source Reference

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASOUR |
| Name: | Source Reference |
| Alias: | MA_SourceReference |
| CamelCase: | sourceReference |
| Data Type: | S121_ResponsibleParty |
| Definition: | The responsible party of the "Source". |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.2.4 |
| Similarity to Source:restyled |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Source Quality

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASODQ |
| Name: | Source Quality |
| Alias: | MA_SourceQuality |
| CamelCase: | sourceQuality |
| Data Type: | DQ_Element |
| Definition: | The quality of the "Source". |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.2.4 |
| Similarity to Source:restyled |  |
| Int1: |  |
| S4: |  |
| Remarks: | This attribute value type is inherited directly from ISO 19115 Metadata and the code list values are described in the ISO document. S100 also used DQ_Element directly from ISO 19115. |
| The DQ_Element attributes are: | "nameOfMeasure", "measureIdentification", "measureDescription", "evaluationMethodType", "evaluationMethodDescription", "evaluationProcedure", "dateTime", "result". |
| Quantity Specifications: |  |

### Source Submission Date

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASOSU |
| Name: | Source Submission Date |
| Alias: | MA_SourceSubmissionDate |
| CamelCase: | sourceSubmissionDate |
| Data Type: | DateTime |
| Definition: | The date of submission of the "Source" by a party |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.2.4 |
| Similarity to Source:restyled |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Source External Archive Identifier

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MAEXAI |
| Name: | Source External Archive Identifier |
| Alias: | MA_ExternalArchiveID |
| CamelCase: | sourceExternalArchiveIdentifier |
| Data Type: | ExtArchive |
| Definition: | The identifier of a "Source" in an external registration. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.2.4 |
| Similarity to Source:restyled |  |
| Int1: |  |
| S4: |  |
| Remarks: | The code list is taken directly for ISO 19152. |
| Quantity Specifications: |  |

### Source Registration Date

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASORC |
| Name: | Source Registration Date |
| Alias: | MA_SourceRecordation |
| CamelCase: | sourceRecordation |
| Data Type: | DateTime |
| Definition: | The date of registration (recordation) of the "Source" by registering authority |
| Reference: | ISO 19152 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Source Document Name

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MASODN |
| Name: | Source Document Name |
| Alias: | MA_SourceDocumentName |
| CamelCase: | sourceDocumentName |
| Data Type: | CharacterString |
| Definition: | Name of the document that defines the object - for example the document (legislation, treaty, title |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Source Online Resource

| Item Type: | Simple Attribute |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MASURL |
| Name: | Source Online Resource |
| Alias: | MA_SourceOnlineResource |
| CamelCase: | sourceOnlineResource |
| Data Type: | onlineResource |
| Definition: | URL - this is official the URL (or equivalent online resource) where the document is distributed. |
| Reference: | ISO 19115 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Source Registry Number

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MASORN |
| Name: | Source Registry Number |
| Alias: | MA_SourceRegistryNumber |
| CamelCase: | sourceRegistryNumber |
| Data Type: | CharacterString |
| Definition: | Unique official identifier of the record in a registry. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | For example, in states with registers of legislative instruments, versioning is controlled by the registry ID. |
| Quantity Specifications: |  |

### Acceptance

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MAACCPT |
| Name: | Acceptance |
| Alias: | MA_Acceptance |
| CamelCase: | acceptance |
| Data Type: | DateTime |
| Definition: | the date of force of law of the source by the authority |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause Annex K.10 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: | This attribute is derived directly from the ExArchive class of ISO 19152. Since the term is so generic it should be restricted to the meaning within the MLB context. |
| Quantity Specifications: |  |

### Data

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MADATA |
| Name: | Data |
| Alias: | MA_ExternalArchiveData |
| CamelCase: | data |
| Data Type: | CharacterString |
| Definition: | The content of the source |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause Annex K.10 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: | This attribute is derived directly from the ExArchive class of ISO 19152. Since the term is so generic it should be restricted to the meaning within the MLB context. The data may include multilingual text per ISO 19115 Annex J (Localization) |
| Quantity Specifications: |  |

### Extraction

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MAEXTR |
| Name: | Extraction |
| Alias: | MA_ExternalArchiveExtraction |
| CamelCase: | extraction |
| Data Type: | DateTime |
| Definition: | The date the source was consulted |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause Annex K.10 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: | This attribute is derived directly from the ExArchive class of ISO 19152. Since the term is so generic it should be restricted to the meaning within the MLB context. |
| Quantity Specifications: |  |

### Recordation

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MARECR |
| Name: | Recordation |
| Alias: | MA_ExternalArchiveRecordation |
| CamelCase: | recordation |
| Data Type: | DateTime |
| Definition: | The date of registration (recordation) of the source by registering authority. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause Annex K.10 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: | This attribute is derived directly from the ExArchive class of ISO 19152. Since the term is so generic it should be restricted to the meaning within the MLB context. |
| Quantity Specifications: |  |

### Submission

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MASUBM |
| Name: | Submission |
| Alias: | MA_ExternalArchiveSubmission |
| CamelCase: | submission |
| Data Type: | DateTime |
| Definition: | The date of submission of the source by a party. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause Annex K.10 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: | This attribute is derived directly from the ExArchive class of ISO 19152. Since the term is so generic it should be restricted to the meaning within the MLB context. |
| Quantity Specifications: |  |

### Online Resource Linkage URL

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MARURL |
| Name: | Online Resource Linkage URL |
| Alias: | MA_OnlineResourceLinkageURL |
| CamelCase: | onlineResourceLinkageURL |
| Data Type: | CharacterString |
| Definition: | Universal Resource Locator (URL) of the online resource. |
| Reference: | ISO 19115 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Online Resource Protocol

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MARPRO |
| Name: | Online Resource Protocol |
| Alias: | MA_Online Resource Protocol |
| CamelCase: | onlineResourceProtocol |
| Data Type: | CharacterString |
| Definition: | Protocol used by the online resource. |
| Reference: | ISO 19115 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Online Resource Application Profile

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MAAPPR |
| Name: | Online Resource Application Profile |
| Alias: | MA_OnlineResourceApplicationProfile |
| CamelCase: | onlineResourceApplicationProfile |
| Data Type: | CharacterString |
| Definition: | Application Profile of the online resource. |
| Reference: | ISO 19115 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Online Resource Name

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MAOLNM |
| Name: | Online Resource Name |
| Alias: | MA_OnlineResourceName |
| CamelCase: | onlineResourceName |
| Data Type: | CharacterString |
| Definition: | Name of the online resource. |
| Reference: | ISO 19115 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Online Resource Description

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MAORDS |
| Name: | Online Resource Description |
| Alias: | MA_OnlineResourceDescription |
| CamelCase: | onlineResourceDescription |
| Data Type: | CharacterString |
| Definition: | Universal Resource Locator (URL) of the online resource. |
| Reference: | ISO 19115 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Online Resource Function

| Item Type: | Simple Attribute |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| AlphaCode: | MAORFN |
| Name: | Online Resource Function |
| Alias: | MA_OnlineResourceFunction |
| CamelCase: | onlineResourceFunction |
| Data Type: | CI_OnlineFunctionCode |
| Definition: | Function of the online resource. |
| Reference: | ISO 19115 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: | The code list CI_OnlineFunctionCode is derived from ISO 19115 metadata. |

## S121 DataTypes

### Area Value

| Item Type: | Data Type |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | LAARAV |
| Name: | LA_AreaValue |
| Alias: | Area Value |
| CamelCase: | areaValue |
| Data Type: |  |
| Definition: | Area of the 2D object |
| Reference: | ISO19152:2012 |
| Definition Source: | ISO19152:2012 clause 6.5.7 |
| Similarity to Source: | Specialization |
| Int1: |  |
| S4: |  |
| Remarks: | Derived directly from ISO 19152 |
| Quantity Specifications: |  |

### Volume Value

| Item Type: | Data Type |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | LAVOLV |
| Name: | LA_VolumeValue |
| Alias: | Volume Value |
| CamelCase: | volumeValue |
| Data Type: |  |
| Definition: | Volume of the 3D object |
| Reference: | ISO19152:2012 |
| Definition Source: | ISO19152:2012 clause 6.5.7 |
| Similarity to Source: | Specialization |
| Int1: |  |
| S4: |  |
| Remarks: | Derived directly from ISO 19152 |
| Quantity Specifications: |  |

### Location Reference

| Item Type: | Data Type |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MALORE |
| Name: | Location Reference |
| Alias: |  |
| CamelCase: | locationReference |
| Data Type: |  |
| Definition: | This data type provides information about a point location. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: |  |

### Online Resource

| Item Type: | Data Type |
| --- | --- |
| Domain: | MLB |
| AlphaCode: | MAOLRS |
| Name: | Online Resource |
| Alias: | MA_OnlineResource |
| CamelCase: | onlineResource |
| Data Type: |  |
| Definition: | This data type provides describes an online resource. |
| Reference: | ISO_19115 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |
| Quantity Specifications: | Online Resource derives from the ISO 19115 metadata standard. |

## S121 Type Lists

### Location Type List

| Item Type: | Code List |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| Name: | Location Type List |
| Alias: | MA_LocationTypeList |
| CamelCase: | locationTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | This code list describes categories of "LocationType" that have a common characteristic related to the legal and administrative aspects of the marine environment. In effect this list consist of all S121 features under the location type classes. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | The code list is registered in the Feature Concept Dictionary as listed values and as such can be expanded to include all aspects of the legal context. |

### Limit Type List

| Item Type: | Code List |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| Name: | Limit Type List |
| Alias: | MA_LimitTypeList |
| CamelCase: | limitTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | This code list describes categories of "LimitType" that have a common characteristic related to the legal and administrative aspects of the marine environment. In effect this list consist of all S121 feature unit limit type classes. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | The code list is registered in the Feature Concept Dictionary as listed values and as such can be expanded to include all aspects of the legal context. |

### Zone Type List

| Item Type: | Code List |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| Name: | Feature Unit Type List |
| Alias: | MA_featureUnitTypeList |
| CamelCase: | featureUnitTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | This code list describes categories of "ZoneType" that have a common characteristic related to the legal and administrative aspects of the marine environment. In effect this list consist of all S121 feature unit zone type classes. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | The code list is registered in the Feature Concept Dictionary as listed values and as such can be expanded to include all aspects of the legal context. |

### Space Type List

| Item Type: | Code List |
| --- | --- |
| Proposal Type: | Addition |
| Domain: | MLB |
| Name: | Space Type List |
| Alias: | MA_SpaceTypeList |
| CamelCase: | spaceTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | This code list describes categories of "SpaceType" that have a common characteristic related to the legal and administrative aspects of the marine environment. In effect this list consist of all S121 feature unit space type classes. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | The code list is registered in the Feature Concept Dictionary as listed values and as such can be expanded to include all aspects of the legal context. |

### Legal Status Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | legalStatusType |
| Name: | Legal Status Type List |
| Alias: | MA_LegalStatusTypeList |
| CamelCase: | legalStatusTypeList |
| Code List Type: | Open EnumerationÂ |
| URI: |  |
| Encoding: |  |
| Definition: | A list of value providing a legislative status for the "FeatureUnit" described. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### Jurisdiction Domain Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | jurisdictionDomainType |
| Name: | Jurisdiction Domain Type List |
| Alias: | MA_JurisdictionDomainTypeList |
| CamelCase: | jurisdictionDomainTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | Defines the vertical juridical domain of the object delimited, based on a breakdown of vertical jurisdictions used in UNCLOS. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | Any particular object may span more than one jurisdiction domain. |

### Releasability Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | releasabilityType |
| Name: | Releasability Type List |
| Alias: | MA_ReleasabilityTypeList |
| CamelCase: | releasabilityTypeList |
| Code List Type: | Open Enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | A list of value providing a releasability status for the "FeatureUnit" described. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### Arc Geometry Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | arcGeometryType |
| Name: | Arc Geometry Type List |
| Alias: | MA_ArcGeometryTypeList |
| CamelCase: | arcGeometryTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | This code list describes the category of lines or arcs that can be represented on the surface of the ellipsoid. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### Spatial Source Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Name: | Spatial Source Type List |
| Alias: | MA_SpatialSourceTypeList |
| CamelCase: | spatialSourceTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | Category of "spatialSourceType". |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.6.6 |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### Administrative Source Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | administrativeSourceType |
| Name: | Administrative Source Type List |
| Alias: | MA_AdministrativeSourceTypeList |
| CamelCase: | administrativeSourceTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | Category of "administrativeSourceType". |
| Reference: | ISO 19152 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | The code list includes: "internationalAgreement", "domesticLegislativeInstrument", "domesticAdministration". |

### Party Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Name: | Party Type List |
| Alias: | MA_PartyTypeList |
| CamelCase: | partyTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | This code list describes categories of "partyType". |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.3.1 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: | The code list is registered in the Feature Concept Dictionary as listed values and as such can be expanded to include all party types. |

### Party Group Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Name: | Party Group Type List |
| Alias: | MA_PartyGroupTypeList |
| CamelCase: | partyGroupTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | This code list describes categories of "partyGroupType". |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.3.2 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: | The code list is registered in the Feature Concept Dictionary as listed values and as such can be expanded to include all administrative types. |

### Basic Administrative Unit Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Name: | Basic Administrative Unit Type List |
| Alias: | MA_ basicAdministrativeUnitTypeList |
| CamelCase: | basicAdministrativeUnitTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | Categories of "basicAdministrativeUnitType". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | The code list includes: "maritimeLimitsAndBoundaries". |

### Legal Status Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | legalStatusType |
| Name: | Legal Status Type List |
| Alias: | MA_LegalStatusTypeList |
| CamelCase: | legalStatusTypeList |
| Code List Type: | Open Enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | A list of values providing a legislative status for the "FeatureUnit" described. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### Restriction Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | restrictionType |
| Name: | Restriction Type List |
| Alias: | MA_RestrictionTypeList |
| CamelCase: | restrictionTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | Category of "restrictionType" |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.9 |
| Similarity to Source:specialization |  |
| Int1: |  |
| S4: |  |
| Remarks: | The code list includes: "timeBasedRestriction", "passageRestriction", "accessRestriction", "useRestriction","jurisdictionRestriction", "resourceRestriction". |

### Right Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Name: | Right Type List |
| Alias: | MA_RightTypeList |
| CamelCase: | rightTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | Category of "rightType" |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | The code list includes: "sovereignty", "sovereignRight", "accessRight", "harvestRight", "easementRight". |

### Responsibility Type List

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | responsibilityType |
| Name: | Responsibility Type List |
| Alias: | MA_ResponsibilityTypeList |
| CamelCase: | responsibilityTypeList |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | Category of "responsibilityType". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### LA Availability Status Type

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | sourceAvailabilityStatus |
| Name: | LA Availability Status Type |
| Alias: | MAAVST |
| CamelCase: | LA_AvailabilityStatusType |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | Category of " sourceAvailabilityStatus ". |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152:2012 clause 6.4.9 |
| Similarity to Source: | specialization |
| Int1: |  |
| S4: |  |
| Remarks: | Derived directly from the ISO 19152 standard. Values are “archiveConverted”, archiveDestroyed”, archiveIncomplete”, “archiveUnknown”, “docAvailable”. |

### LA Surface Relation Type

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | MASURT |
| Name: | LA Surface Relation Type |
| Alias: | MA_surfaceRelationType |
| CamelCase: | surfaceRelationType |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | This data type provides information about the relationship to the surface. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152 clause 6.5.8 |
| Similarity to Source: | identical |
| Int1: |  |
| S4: |  |
| Remarks: | This codelist is taken directly from ISO 19152 |

### LA Volume Type

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | MAVOLT |
| Name: | LA Volume Type |
| Alias: | MA_volumeType |
| CamelCase: | volumeType |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | Description of the type of volume. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152 clause 6.5.8 |
| Similarity to Source: | generalization |
| Int1: |  |
| S4: |  |
| Remarks: | This codelist is taken directly from ISO 19152 |

### LA Area Type

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | MAARTY |
| Name: | LA Area Type |
| Alias: | MA_areaType |
| CamelCase: | areaType |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | Description of the type of volume. |
| Reference: | ISO 19152 |
| Definition Source: | ISO 19152 clause 6.5.8 |
| Similarity to Source: | generalization |
| Int1: |  |
| S4: |  |
| Remarks: | This codelist is taken directly from ISO 19152 |

### LA Interpolation Type

| Item Type: | Code List |
| --- | --- |
| Domain | ISO 19152 |
| Associated Attribute: | arcGeometry |
| Name | Arc Interpolation Type |
| Alias | MA_InterpolationTypeList |
| Definition | Interpolation Types |
| camelCaseCode | LA_InterpolationType |
| Remarks | This code list derives directly from ISO 19152 |
| Use Type | Marine Administrative Domain |
| Code List Type | Closed Enumeration (midArc, start, end, mid, isolated) |
| URI |  |
| Encoding |  |
| References | ISO 19152 |

### LA Point Type

| Item Type: | Code List |
| --- | --- |
| Domain | ISO 19152 |
| Associated Attribute: | pointType |
| Name | Point Type |
| Alias | MA_PointType |
| Definition | Point Types. |
| camelCaseCode | LA_PointType |
| Remarks | This code list derives directly from ISO 19152 |
| Use Type | Marine Administrative Domain |
| Code List Type | Closed Enumeration (source, control, noSource) |
| URI |  |
| Encoding |  |
| References | ISO 19152 |

### CI_OnlineFunctionCode

| Item Type: | Code List |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | ronlineResourceFunction |
| Name: | CI_OnlineFunctionCode |
| Alias: | MA_CI_OnlineFunctionCode |
| CamelCase: | onlineFunctionCode |
| Code List Type: | open enumeration |
| URI: |  |
| Encoding: |  |
| Definition: | Function performed by the resource |
| Reference: | ISO 19115 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: | Derived directly from the ISO 19115 standard. Values are “download”, information”, offlineAccess”, “order”, “search”. |

## S121 Listed Values

### -- Legal Status Type List

#### Lodged

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | legalStatusTypeList |
| Name: | Lodge |
| Alias: |  |
| CamelCase: | deposited |
| Definition: | Describes a "FeatureUnit" that is deposited or lodged with the United Nations for purposes related to UNCLOS. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Signed

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | legalStatusTypeList |
| Name: | Signed |
| Alias: |  |
| CamelCase: | signed |
| Definition: | Describes a "FeatureUnit" that is signed by the intersted party or parties. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Proposed

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | legalStatusTypeList |
| Name: | Proposed |
| Alias: |  |
| CamelCase: | proposed |
| Definition: | Describes a "FeatureUnit" that is proposed between the intersted party or parties. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Predicated

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | legalStatusTypeList |
| Name: | Predicated |
| Alias: |  |
| CamelCase: | predicated |
| Definition: | Describes a "FeatureUnit" that is unitalerally declared, affirmed and asserted. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Development

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | legalStatusTypeList |
| Name: | Development |
| Alias: |  |
| CamelCase: | development |
| Definition: | Describes a "FeatureUnit" that is in development. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### In force

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | legalStatusTypeList |
| Name: | In force |
| Alias: |  |
| CamelCase: | inForce |
| Definition: | Describes a "FeatureUnit" that is effective, enforceable, that has legal effect, legal force and is operative. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Releasability Type List

#### Official

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | releasabilityTypeList |
| Name: | Official |
| Alias: |  |
| CamelCase: | official |
| Definition: | Describes a "FeatureUnit" that can be officially released. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Internal

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | releasabilityTypeList |
| Name: | Internal |
| Alias: |  |
| CamelCase: | internal |
| Definition: | Describes a "FeatureUnit" that can only be released internally |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Controlled

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | releasabilityTypeList |
| Name: | Controlled |
| Alias: |  |
| CamelCase: | controlled |
| Definition: | Describes a "FeatureUnit" that bears security restrictions. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- LA_SurfaceRelationType

#### Mixed

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_surfaceRelationType |
| Name: | Mixed |
| Alias: |  |
| CamelCase: | mixed |
| Definition: | Relationship of spatial unit to surface. Mixed relationship. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Below

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_surfaceRelationType |
| Name: | Below |
| Alias: |  |
| CamelCase: | below |
| Definition: | Relationship of spatial unit to surface. Below Surface. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Above

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_surfaceRelationType |
| Name: | Above |
| Alias: |  |
| CamelCase: | above |
| Definition: | Relationship of spatial unit to surface. Above surface. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### On Surface

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_surfaceRelationType |
| Name: | On Surface |
| Alias: |  |
| CamelCase: | onSurface |
| Definition: | Relationship of spatial unit to surface. On surface. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Basic Administrative Unit Type List

#### Maritime Limits and Boundaries

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | basicAdministrativeUnitTypeList |
| Name: | Maritime Limits and Boundaries |
| Alias: |  |
| CamelCase: | maritimeLimitsAndBoundaries |
| Definition: | Administrative Unit relates to "Maritime Limits and Boundaries" |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Location Type List

#### Location

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | locationTypeList |
| Name: | Location |
| Alias: |  |
| CamelCase: | location |
| Definition: | Identifier that a Feature Unit is of type generic Location |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Baseline Point

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | locationTypeList |
| Name: | Baseline Point |
| Alias: |  |
| CamelCase: | baselinePoint |
| Definition: | Identifier that a Feature Unit is of type Baseline Point |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Contributing Point

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | locationTypeList |
| Name: | Contributing Point |
| Alias: |  |
| CamelCase: | contributingPoint |
| Definition: | Identifier that a Feature Unit is of type Contributing Point |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Limit Point

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | locationTypeList |
| Name: | Limit Point |
| Alias: |  |
| CamelCase: | limitPoint |
| Definition: | Identifier that a Feature Unit is of type Limit Point |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Boundary Point

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | locationTypeList |
| Name: | Boundary Point |
| Alias: |  |
| CamelCase: | boundaryPoint |
| Definition: | Identifier that a Feature Unit is of type Boundary Point |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Limit Type List

#### Limit

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Limit |
| Alias: |  |
| CamelCase: | limit |
| Definition: | Identifier that a Feature Unit is of type generic Limit |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### International Boundary

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | International Boundary |
| Alias: |  |
| CamelCase: | internationalBoundary |
| Definition: | Identifier that a Feature Unit is of type International Boundary |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Baseline

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Baseline |
| Alias: |  |
| CamelCase: | baseline |
| Definition: | Identifier that a Feature Unit is of type Baseline |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Archipelagic Baseline

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Archipelagic Baseline |
| Alias: |  |
| CamelCase: | archipelagicBaseline |
| Definition: | Identifier that a Feature Unit is of type Archipelagic Baseline |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Bay Baseline

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Bay Baseline |
| Alias: |  |
| CamelCase: | bayBaseline |
| Definition: | Identifier that a Feature Unit is of type Bay Baseline |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Normal Baseline

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Normal Baseline |
| Alias: |  |
| CamelCase: | normalBaseline |
| Definition: | Identifier that a Feature Unit is of type Normal Baseline |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Straight Baseline

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Straight Baseline |
| Alias: |  |
| CamelCase: | straightBaseline |
| Definition: | Identifier that a Feature Unit is of type Straight Baseline |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Low Tide Elevation Baseline

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Low Tide Elevation Baseline |
| Alias: |  |
| CamelCase: | lowTideElevationBaseline |
| Definition: | Identifier that a Feature Unit is of type Low Tide Elevation Baseline |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Mouth of River Baseline

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Mouth of River Baseline |
| Alias: |  |
| CamelCase: | mouthOfRiverBaseline |
| Definition: | Identifier that a Feature Unit is of type Mouth of River Baseline |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Port Baseline

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Port Baseline |
| Alias: |  |
| CamelCase: | portBaseline |
| Definition: | Identifier that a Feature Unit is of type Port Baseline |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Reef Baseline

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Reef Baseline |
| Alias: |  |
| CamelCase: | reefBaseline |
| Definition: | Identifier that a Feature Unit is of type Reef Baseline |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Outer Limit of the Territorial Sea

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Outer Limit of the Territorial Sea |
| Alias: |  |
| CamelCase: | outerLimitOfTheTerritorialSea |
| Definition: | Identifier that a Feature Unit is of type Outer Limit of the Territorial Sea |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Outer Limit of the Contiguous Zone

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Outer Limit of the Contiguous Zone |
| Alias: |  |
| CamelCase: | outerLimitOfTheContiguousZone |
| Definition: | Identifier that a Feature Unit is of type Outer Limit of the Contiguous Zone |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Outer Limit of the Exclusive Economic Zone

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Outer Limit of the Exclusive Economic Zone |
| Alias: |  |
| CamelCase: | outerLimitOfTheExclusiveEconomicZone |
| Definition: | Identifier that a Feature Unit is of type Outer Limit of the Exclusive Economic Zone |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Outer Limit of the Continental Shelf

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Outer Limit of the Continental Shelf |
| Alias: |  |
| CamelCase: | outerLimitOfTheContinentalShelf |
| Definition: | Identifier that a Feature Unit is of type Outer Limit of the Continental Shelf |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Outer Limit of the Roadstead

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Outer Limit of the Roadstead |
| Alias: |  |
| CamelCase: | outerLimitOfTheRoadstead |
| Definition: | Identifier that a Feature Unit is of type Outer Limit of the Roadstead |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Construction Line

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | limitTypeList |
| Name: | Construction Line |
| Alias: |  |
| CamelCase: | constructionLine |
| Definition: | Identifier that a Feature Unit is of type Construction Line |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Zone Type List

#### Zone

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | zoneTypeList |
| Name: | Zone |
| Alias: |  |
| CamelCase: | zone |
| Definition: | Identifier that a Feature Unit is of type generic Zone |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Internal Water

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | zoneTypeList |
| Name: | Internal Water |
| Alias: |  |
| CamelCase: | internalWater |
| Definition: | Identifier that a Feature Unit is of type Internal Water |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Archipelagic Water

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | zoneTypeList |
| Name: | Archipelagic Water |
| Alias: |  |
| CamelCase: | archipelagicWater |
| Definition: | Identifier that a Feature Unit is of type Archipelagic Water |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Territorial Sea

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | zoneTypeList |
| Name: | Territorial Sea |
| Alias: |  |
| CamelCase: | territorialSea |
| Definition: | Identifier that a Feature Unit is of type Territorial Sea |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Roadstead

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | zoneTypeList |
| Name: | Roadstead |
| Alias: |  |
| CamelCase: | roadstead |
| Definition: | Identifier that a Feature Unit is of type Roadstead |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Contiguous Zone

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | zoneTypeList |
| Name: | Contiguous Zone |
| Alias: |  |
| CamelCase: | contiguousZone |
| Definition: | Identifier that a Feature Unit is of type Contiguous Zone |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Exclusive Economic Zone

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | zoneTypeList |
| Name: | Exclusive Economic Zone |
| Alias: |  |
| CamelCase: | exclusiveEconomicZone |
| Definition: | Identifier that a Feature Unit is of type Exclusive Economic Zone |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Continental Shelf

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | zoneTypeList |
| Name: | Continental Shelf |
| Alias: |  |
| CamelCase: | continentalShelf |
| Definition: | Identifier that a Feature Unit is of type Continental Shelf |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### High Sea

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | zoneTypeList |
| Name: | High Sea |
| Alias: |  |
| CamelCase: | highSea |
| Definition: | Identifier that a Feature Unit is of type High Sea |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### The Area

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | zoneTypeList |
| Name: | The Area |
| Alias: |  |
| CamelCase: | theArea |
| Definition: | Identifier that a Feature Unit is of type The Area |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Space Type List

#### Space

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | spaceTypeList |
| Name: | Space |
| Alias: |  |
| CamelCase: | space |
| Definition: | Identifier that a Feature Unit is of type generic Space |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Interpolation Type List

#### Deflection

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | interpolationTypeList |
| Name: | Deflection |
| Alias: |  |
| CamelCase: | deflection |
| Definition: | A turning point along a line. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Densification

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | interpolationTypeList |
| Name: | Densification |
| Alias: |  |
| CamelCase: | Densification |
| Definition: | Describes a location that densifies a line to ensure the geometry of a feature is correctly represented. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Point Type List

#### Defined

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | pointTypeList |
| Name: | Defined |
| Alias: |  |
| CamelCase: | defined |
| Definition: | Describes a "Location" that is derived from a legislative document or other definitive sources. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Computed

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | pointTypeList |
| Name: | Computed |
| Alias: |  |
| CamelCase: | computed |
| Definition: | Describes a location that is computed in accordance with the definition described in the source through proper geodetic calculations; for example, the intersection of two arcs over an ellipsoidal surface. A point may be established to support construction computations. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Arc Geometry Type List

#### Geodesic

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | arcGeometryTypeList |
| Name: | Geodesic |
| Alias: |  |
| CamelCase: | geodesic |
| Definition: | Describes a path of shortest distance along the surface of an ellipsoid, namely a segment of a great circle. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Loxodrome

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | arcGeometryTypeList |
| Name: | Loxodrome |
| Alias: |  |
| CamelCase: | loxodrome |
| Definition: | Describes an arc crossing all meridians of longitude at the same angle; a path with constant bearing. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Jurisdiction Domain Type List

#### Airspace

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | jurisdictionDomainTypeList |
| Name: | Airspace |
| Alias: |  |
| CamelCase: | airspace |
| Definition: | Describes a space in the air above the "landSurface" or "waterSurface". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Land Surface

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | jurisdictionDomainTypeList |
| Name: | Land Surface |
| Alias: |  |
| CamelCase: | landSurface |
| Definition: | Describe the interface between "subsoil" and "airspace" |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Water Surface

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | jurisdictionDomainTypeList |
| Name: | Water Surface |
| Alias: |  |
| CamelCase: | waterSurface |
| Definition: | Describe the interface between the "airspace" and "waterColumn" |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Water Column

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | jurisdictionDomainTypeList |
| Name: | Water Column |
| Alias: |  |
| CamelCase: | waterColumn |
| Definition: | Describes a space from the "seabedSurface" up to the "waterSurface". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Seabed Surface

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | jurisdictionDomainTypeList |
| Name: | Seabed Surface |
| Alias: |  |
| CamelCase: | seabedSurface |
| Definition: | Describes the interface between the "waterColumn" and the "subsoil", submerged land surface found under the oceans. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Subsoil

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | jurisdictionDomainTypeList |
| Name: | Subsoil |
| Alias: |  |
| CamelCase: | subsoil |
| Definition: | Describes a space below the "landSurface" and the "seabedSurface". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Administrative Source Type List

#### International Agreement

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | administrativeSourceTypeList |
| Name: | International Agreement |
| Alias: |  |
| CamelCase: | internationalAgreement |
| Definition: | Describes an international agreement for example: Treaty, Agreement, MOU (Memorandum of Understanding), Exchange of letters. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Domestic Legislative Instrument

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | administrativeSourceTypeList |
| Name: | Domestic Legislative Instrument |
| Alias: |  |
| CamelCase: | domesticLegislativeInstrument |
| Definition: | Defines a source describing a primary and secondary legislative processes and domestic implementations of treaties. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Domestic Administrative

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | administrativeSourceTypeList |
| Name: | Domestic Administrative |
| Alias: |  |
| CamelCase: | domesticAdministrative |
| Definition: | Defines a source that is based under a legislative framework by the authority given under domestic legislation; for example, petroleum permits. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Spatial Source Type List

#### Field Sketch

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | spatialSourceTypeList |
| Name: | Field Sketch |
| Alias: |  |
| CamelCase: | fieldSketch |
| Definition: | Source data collected from hydrographer's or mapper's field sketch |
| Reference: | ISO 19152 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### GNSS Survey

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | SpatialSourceTypeList |
| Name: | GNSS Survey |
| Alias: |  |
| CamelCase: | gnssSurvey |
| Definition: | Source data collected from GNSS satellite positioning |
| Reference: | ISO 19152 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Relative Measurement

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | SpatialSourceTypeList |
| Name: | Relative Measurement |
| Alias: |  |
| CamelCase: | relativeMeasurement |
| Definition: | Source data collected based on relative measurement |
| Reference: | ISO 19152 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Orthographic Photo

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | SpatialSourceTypeList |
| Name: | Orthographic Photo |
| Alias: |  |
| CamelCase: | orthoPhoto |
| Definition: | Source data collected base on "Orthographic Photo" |
| Reference: | ISO 19152 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Satellite Imagery

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | SpatialSourceTypeList |
| Name: | Satellite Imagery |
| camelCaseCode: | SatelliteImagery |
| Definition: | Source data collected base on "Satellite Imagery". |
| Reference: | ISO 19152 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Topographic Map

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | SpatialSourceTypeList |
| Name: | Topographic Map |
| Alias: |  |
| CamelCase: | topographicMap |
| Definition: | Source data collected base on "Topographic Map" data |
| Reference: | ISO 19152 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### LiDAR Survey

| tem Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | SpatialSourceTypeList |
| Name: | LiDAR Survey |
| Alias: |  |
| CamelCase: | lidarSurvey |
| Definition: | Source data collected from a "LiDAR Survey". |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Video

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | SpatialSourceTypeList |
| Name: | Video |
| Alias: |  |
| CamelCase: | video |
| Definition: | Source data collected base on "Video" information. |
| Reference: | ISO 19152 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Hydrographic Chart

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | SpatialSourceTypeList |
| Name: | Hydrographic Chart |
| Alias: |  |
| CamelCase: | hydrographicChart |
| Definition: | Source data collected base on "Hydrographic Chart" data. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Party Type List

#### International Organisation

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | partyTypeList |
| Name: | International Organisation |
| Alias: |  |
| CamelCase: | internationalOrganisation |
| Definition: | Identifies party as being an International Organization. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### State Country

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | partyTypeList |
| Name: | StateCountry |
| Alias: |  |
| CamelCase: | stateCountry |
| Definition: | Identifies party as being a State or Country. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Province State

| Item Type: | Code List Value |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| Associated Attribute: | partyTypeList |
| Name: | Province State |
| Alias: |  |
| CamelCase: | provinceState |
| Definition: | Identifies party as being a province or state. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Non Natural Person

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | partyTypeList |
| Name: | Non Natural Person |
| Alias: |  |
| CamelCase: | nonNaturalPerson |
| Definition: | Identifies party as being a non-natural person such as a corporation. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Natural Person

| Item Type: | Code List Value |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| Associated Attribute: | partyTypeList |
| Name: | Natural Person |
| Alias: |  |
| CamelCase: | naturalPerson |
| Definition: | Defines a natural person natural person |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Party Group Type List

#### Agreement

| Item Type: | Code List Value |
| --- | --- |
| Proposal Type: |  |
| Domain: | MLB |
| Associated Attribute: | groupPartyTypeList |
| Name: | Agreement |
| Alias: |  |
| CamelCase: | agreement |
| Definition: | The resulting group consists of parties related to an agreement. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Association

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | partyGroupTypeList |
| Name: | Association |
| Alias: |  |
| CamelCase: | association |
| Definition: | The resulting group is an association. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Restriction Type List

#### Access Restriction

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | restrictionTypeList |
| Name: | Access Restriction |
| Alias: |  |
| CamelCase: | accessRestriction |
| Definition: | A restriction on the right of access. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Jurisdiction Restriction

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | restrictionTypeList |
| Name: | Jurisdiction Restriction |
| Alias: |  |
| CamelCase: | jurisdictionRestriction |
| Definition: | A restriction on jurisdiction right. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Passage Restriction

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | restrictionTypeList |
| Name: | Passage Restriction |
| Alias: |  |
| CamelCase: | passageRestriction |
| Definition: | A restriction on the right of access for passage. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Resource Restriction

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | restrictionTypeList |
| Name: | Resource Restriction |
| Alias: |  |
| CamelCase: | resourceRestriction |
| Definition: | A restriction on the right of harvest of a resource. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Time Based Restriction

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | restrictionTypeList |
| Name: | Time Based Restriction |
| Alias: |  |
| CamelCase: | timeBasedRestriction |
| Definition: | A restriction on any right based on time. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Use Restriction

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | restrictionTypeList |
| Name: | Use Restriction |
| Alias: |  |
| CamelCase: | useRestriction |
| Definition: | A restriction on usage right. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Right Type List

#### Sovereignty

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | rightTypeList |
| Name: | Sovereignty |
| Alias: |  |
| CamelCase: | sovereignty |
| Definition: | Absolute prescriptive and enforcement power, limited only by coastal state international obligations. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Sovereign Right

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | rightTypeList |
| Name: | Sovereign Right |
| Alias: |  |
| CamelCase: | sovereignRight |
| Definition: | The right of an exclusivity of jurisdiction. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Access Right

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | rightTypeList |
| Name: | Access Right |
| Alias: |  |
| CamelCase: | accessRight |
| Definition: | The right of access including passage. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Easement Right

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | rightTypeList |
| Name: | Easement Right |
| Alias: |  |
| CamelCase: | easementRight |
| Definition: | The right to establish infrastructure. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Harvest Right

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | rightTypeList |
| Name: | Harvest Right |
| Alias: |  |
| CamelCase: | harvestRight |
| Definition: | The right to harvest a marine resource such as fishing, mineral mining or oil. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Contiguous Right

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | rightTypeList |
| Name: | Contiguous Right |
| Alias: |  |
| CamelCase: | contiguousRight |
| Definition: | A right on the contiguous zone. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- Responsibility Type List

#### Maintenance Responsibility

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | responsibilityTypeList |
| Name: | Maintenance Responsibility |
| Alias: |  |
| CamelCase: | maintenanceResponsibility |
| Definition: | Defines a responsibility to maintain an administrative unit. |
| Reference: | ISO 19152 |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- LA Availability Status Type

#### Archive Converted

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_AvailabilityStatusType |
| Name: | Archive Converted |
| Alias: |  |
| CamelCase: | archiveConverted |
| Definition: | Source archive has been converted. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Archive Destroyed

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_AvailabilityStatusType |
| Name: | Archive Destroyed |
| Alias: |  |
| CamelCase: | archiveDestroyed |
| Definition: | Source archive has been destroved. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Archive Incomplete

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_AvailabilityStatusType |
| Name: | Archive Incomplete |
| Alias: |  |
| CamelCase: | archiveIncomplete |
| Definition: | Source archive I incomplete. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Archive Unknown

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_AvailabilityStatusType |
| Name: | Archive Unknown |
| Alias: |  |
| CamelCase: | archiveUnknown |
| Definition: | Source archive is unknown. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Document Available

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_AvailabilityStatusType |
| Name: | Document Available |
| Alias: |  |
| CamelCase: | docAvailable |
| Definition: | Source document is available. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- LA Area Type

#### Official Area

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_AreaType |
| Name: | Official Area |
| Alias: |  |
| CamelCase: | officialArea |
| Definition: | Area has been officially established. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Non Official Area

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_AreaType |
| Name: | Non-official Area |
| Alias: |  |
| CamelCase: | nonOfficialArea |
| Definition: | Area has not been officially established. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Calculated Area

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_AreaType |
| Name: | Calculated Area |
| Alias: |  |
| CamelCase: | calculatedArea |
| Definition: | Area has been established through a calculation. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Surveyed Area

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_AreaType |
| Name: | Surveyed Area |
| Alias: |  |
| CamelCase: | surveyedArea |
| Definition: | Area has been established through use of a survey technique. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

### -- LA Volume Type

#### Official Volume

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_VolumeType |
| Name: | Official Volume |
| Alias: |  |
| CamelCase: | officialVolume |
| Definition: | Volume has been officially established. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Non Official Volume

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_VolumeType |
| Name: | Non-official Volume |
| Alias: |  |
| CamelCase: | nonOfficialVolume |
| Definition: | Volume has not been officially established. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Calculated Volume

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_VolumeType |
| Name: | Calculated Volume |
| Alias: |  |
| CamelCase: | calculatedVolume |
| Definition: | Volume has been established through a calculation. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

#### Surveyed Volume

| Item Type: | Code List Value |
| --- | --- |
| Domain: | MLB |
| Associated Attribute: | LA_VolumeType |
| Name: | Surveyed Volume |
| Alias: |  |
| CamelCase: | surveyedVolume |
| Definition: | Volume has been established through use of a survey technique. |
| Reference: |  |
| Definition Source: |  |
| Similarity to Source: |  |
| Int1: |  |
| S4: |  |
| Remarks: |  |

IHO Dictionary, S-32, 5th Edition, 390

IHO Dictionary, S-32, 5th Edition, 393